SE GT SU

Kaz Power Plant Upgrade Project — Job Hazard Analysis (JHA)

| Document No: | 1 | Rev: | 1 (Corrected) | Date: | 14.09.2026 |
| --- | --- | --- | --- | --- | --- |
| Subcontractor: | KAZ | Personnel Tot.: | 4 (to confirm) | Prepared by: | ■■ TO BE COMPLETED ■■ |
| Activity: | Kaz Power Plant Upgrade Project — Construction, Installation, Test & Commissioning of the ■■ TO BE COMPLETED ■■ (corrected spelling of “Constraction, instalation, commitiong of SS extantion”; confirm the exact extension scope: GT / Substation / Steam Supply) |  |  |  |  |
| Location: | Basrah, Iraq | Work Area: | Khur Al Zubair Power Station | Client / Owner: | ■■ TO BE CONFIRMED (e.g. MOE / BGC) ■■ |
| Used Vehicles: | Truck, Excavator (corrected: Manlift & Crane reclassified as equipment) | Used Equipment’s: | Hand Tools, Power Tools, Lifting Equipment’s, Test Equipment’s, Manlift, Crane |  |  |
| Team Members: | Uğur Akbulut, Onur Özvatan, Abdurrahman Öğüt, Doğukan Arandı (confirm: the same persons appear as approvers in Step 4 — see independence note) |  |  |  |  |

NOTE: If after risk evaluation and with all the necessary controls in place the risk rating (R) is HIGH (13–25), the EHS department is to be contacted for guidance on the task/process.

EMERGENCY CONTACTS (local numbers were missing in Rev 0 — complete before approval)

| EMERGENCY CONTACT NUMBER | POLICE | FIRE STATION | AMBULANCE | NEAREST HOSPITAL (new column) |
| --- | --- | --- | --- | --- |
| SIEMENS ENERGY EMERGENCY: +90 533 625 9494 (existing — confirm still valid) | 112 (generic) — local station number: ■■ TO BE COMPLETED ■■ | ■■ TO BE COMPLETED ■■ local station number | ■■ TO BE COMPLETED ■■ local station number | ■■ TO BE COMPLETED ■■ name, phone & response time (also used in row 31) |

REVISION NOTES — Rev 1 (14.09.2026)

1. Severity scale corrected: level 4 defined (Major Injury / Permanent Disability).

2. Risk matrix bands corrected: 1–7 Low / 8–12 Medium / 13–25 High (score 7 was previously unclassified).

3. Residual-risk logic corrected: controls now reduce LIKELIHOOD (primary) and severity only where the worst outcome is genuinely eliminated — previously severity was mechanically set to 1 in every row and residual scores were all identical (R=4).

4. Duplicate rows removed (old 11=12, 14=15); mismatched precautions fixed (old rows 7, 14, 15, 21); template remnants (&, stray numbers) removed.

5. Management processes (STOP, Incident Management) removed from the risk matrix — tracked as EHS program KPIs, not hazards.

6. Responsibility corrected in HV electrical work: controls implemented by the executing contractor with plant-operations coordination (was “by the customer”).

7. Missing site risks added: heat stress, security, live-plant interface, critical lifts, gas commissioning, hot surfaces, oil/chemical spill, noise, dust/fumes, moving machinery, ergonomics, chemicals, medical evacuation, falling objects.

8. PTW/permit reference added per row; emergency contact table completed with placeholders pending local data.

9. Typos corrected (spelling, “SS extantion”, “POTANTIAL”, “VOLATILES”→vectors, “Panzer”→vaccination, EN136 mislabeled as dust mask → FFP3 EN149).

10. Step 3 PPE matrix now references specific risk numbers instead of blanket “All”.

STEP 1 — RISK GROUPS (26) & RELATED WORK PERMITS

| No | Risk Group | Relevant Work Permit | Relevant to this JHA? (corrected ticks) |
| --- | --- | --- | --- |
| 1 | Electric shock or exposure to electric arc | ELECTRICAL WORK PERMIT | X |
| 2 | Fall from height | WORK AT HEIGHT PERMIT | X |
| 3 | Falling object from height | WORK AT HEIGHT PERMIT | X |
| 4 | Hot work (oxy-LPG, oxy-acetylene, LPG, torch) | HOT WORK PERMIT | X* (apply during installation/welding phase — confirm scope) |
| 5 | Welding (electrode, aluminum) | HOT WORK PERMIT | X* (apply during installation/welding phase — confirm scope) |
| 6 | Ergonomic strain / manual handling | JHA | X |
| 7 | Hazardous waste | MOTAT Records | X |
| 8 | Use of mobile electrical panel and extension cable | ELECTRICAL WORK PERMIT | X |
| 9 | Use of crane, hoist, forklift, compressor, personal carrier, etc. | CRANE WORK PERMIT, LIFTING PLAN | X |
| 10 | Use of construction machinery (excavator, truck) | EXCAVATION WORK PERMIT | X |
| 11 | Angle grinder use | JHA | X |
| 12 | Use of chemical substances (ethyl alcohol, acid, base, epoxy, etc.) | JHA | X |
| 13 | Waste generation due to activity (cable, metal, paper, hydrocarbon, etc.) | MOTAT Records | X |
| 14 | Explosive areas | JHA | — (none identified — confirm) |
| 15 | Exposure to dust, vapor, gas, welding fumes, chemical & biological agents | JHA | X |
| 16 | Moving machine parts or work equipment (entrapment, impact, etc.) | JHA | X |
| 17 | Slip and trip hazards on the ground | JHA | X |
| 18 | Contact with extremely cold or hot surfaces | JHA | X (ADDED — turbine/piping during commissioning) |
| 19 | Working near energized open busbars and conductors in low-voltage systems | ELECTRICAL WORK PERMIT | X |
| 20 | Working in high-voltage system (≥1000 V) | ELECTRICAL WORK PERMIT, EKAD | X |
| 21 | Environmental factors, working in extremely hot & cold climate | JHA | X (ADDED — Basra summer up to ~50°C) |
| 22 | Hand tool use (hammer, cutting tools, etc.) | JHA | X |
| 23 | Use of scaffolding, ladder and platform | JHA → WORK AT HEIGHT PERMIT | X |
| 24 | Insufficient lighting and ventilation | CONFINED SPACE WORK PERMIT | X |
| 25 | Working in confined or restricted space | CONFINED SPACE WORK PERMIT | X |
| 26 | Use of pressure vessel | CONFINED SPACE WORK PERMIT | — (confirm applicability) |

STEP 2 — RISK ASSESSMENT (corrected)

HAZARD / POTENTIAL RISK GUIDE — see Step 1 table for the corrected tick marks.

Severity (S) — corrected scale:  1: No Injury   2: Minor Injury (First Aid)   3: Lost Time Injury   4: Major Injury / Permanent Disability (ADDED — was undefined)   5: Death

Likelihood (L):  1: Very Unlikely   2: Unlikely   3: Likely   4: Very Likely   5: Certain

Risk Score bands (corrected):  1–7 = LOW (Acceptable, Monitor & Review)   8–12 = MEDIUM (Acceptable With Controls)   13–25 = HIGH (Unacceptable — EHS involvement required)    [score 7 was previously unclassified]

| No | Pathway of activity / hazard Precautions (corrected) |  |  |  |  |  | Post-control assessment (with controls in place) Permit (PTW) reference |  |  |  |  | Status (Rev 1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | S | R | Rating |  | L | S | R | Rating |  |  |
| 1 | Pre-Work Compliance & Mobilization | 2 | 3 | 6 | Low | All personnel: valid WORK visa (corrected from “assembly visa”); medical certificate <12 months; vocational qualifications verified (crane, forklift, welder, scaffold erector, electrician, maintenance); site + SE EHS field induction completed with signed attendance; emergency plan understood (nearest hospital + response time); SE and client risk assessments read; PPE issued per Step 3; working hours: max 11 h day / 7.5 h night, 1 rest day per 7 days, overtime ≤270 h/year; keep passport + local cash for contingencies. | 1 | 3 | 3 | Low | Checklist (not a PTW) | Corrected (L 4→2; also a mandatory checklist) |
| 2 | General Site Work | 3 | 3 | 9 | Medium | Complete LMRA before each task; obtain the required work permit from the client (PTW authority: ■■ TO BE COMPLETED ■■ — open item per PTW review); report observed risks to the client and request action; escalate to Project Manager if no action is taken. | 2 | 3 | 6 | Low | Per task (LMRA + permits) | Corrected |
| 3 | Driving & Transport to Site | 3 | 4 | 12 | Medium | Valid driver’s license for the vehicle type + medical fitness; journey planning (distance, road conditions, weather); obey traffic rules and speed limits; seat belt at all times; no mobile phone (incl. hands-free); no driving fatigued — break every 2 h; loads within weight limits and secured; daily vehicle inspection; no night travel without approval; tires suitable for Basrah conditions (summer heat / monsoon). | 2 | 3 | 6 | Low | N/A | Corrected (S 3→4: fatal road-accident potential in Iraq; seasonal text localized) |
| 4 | Unrestricted / Unbarricaded Area | 3 | 3 | 9 | Medium | Barricade the work area where possible; keep personnel who are not required out of the hazardous area; signage; trained and qualified personnel only. | 2 | 3 | 6 | Low | N/A | Corrected (old row 16) |
| 5 | Unauthorized Persons / Other Subcontractors in Work Area | 3 | 3 | 9 | Medium | Access control (ID badges); induction for every visitor/subcontractor; daily coordination meeting; demarcate zones; inform other subcontractors of cross-activity risks. | 2 | 3 | 6 | Low | Access / induction records | Corrected (old row 20) |
| 6 | Working at Height — Fall from Height | 2 | 5 | 10 | Medium | Work at height permit; scaffolding/working platform inspected and approved (tagged) before access; EN 361 full-body harness with proper anchor point; guardrails where possible; rescue plan for a suspended person. | 1 | 3 | 3 | Low | WORK AT HEIGHT PERMIT | Corrected (old rows 11+12 merged — duplicate removed; S 5→3 justified: harness converts a fatal fall to a survivable one) |
| 7 | Falling Object from Height | 3 | 4 | 12 | Medium | Tool lanyards; toe boards on platforms; exclusion zone below work; materials stacked and secured; signage; no work over occupied areas without protection. | 2 | 3 | 6 | Low | WORK AT HEIGHT PERMIT | NEW (risk group 3 ticked in Step 1 but had no row in Rev 0) |
| 8 | Slips, Trips & Falls on Same Level | 3 | 2 | 6 | Low | Cable ramps / cables routed outside the hazard zone; remove unneeded materials and tools; keep traffic routes clear; housekeeping; check floor for deformation, lubrication or gaps; non-slip footwear. | 2 | 2 | 4 | Low | N/A | Corrected (old row 13; S 3→2) |
| 9 | Lifting Operations (crane, hoist, forklift) | 3 | 4 | 12 | Medium | Crane Work Permit + Lifting Plan; certified operator + rigger + banksman; SWL respected (never exceed); slings/chains visually inspected with valid tag; exclusion zone under load — never stand under a load; spotter; weather limit (wind < ■■ TO BE COMPLETED ■■ km/h — reference client standard); first-aid kit + extinguisher in vehicles. | 2 | 4 | 8 | Medium | CRANE WORK PERMIT + LIFTING PLAN | Corrected (old row 7 split: title now matches precautions; residual stays MEDIUM) |
| 10 | Critical / Heavy Lifts (turbine module, generator, transformer) | 2 | 5 | 10 | Medium | Separate CRITICAL LIFT PLAN: lift engineer appointed; load test; crane capacity and ground bearing verified; lifting director; dedicated exclusion zone; client hold point before each critical lift; ■■ TO BE COMPLETED ■■ confirm list of critical lifts. | 1 | 4 | 4 | Low | LIFTING PLAN (critical) | NEW |
| 11 | Earthworks & Excavation | 3 | 4 | 12 | Medium | Excavation Permit; UTILITY LOCATE / SCAN before any digging — no excavation over underground services (cables, gas/steam lines) without a scan; shoring or sloping above 1.2 m depth; spoil kept off the edge; observer for equipment maneuvering; equipment licenses, driver documents + on-the-job training. | 2 | 3 | 6 | Low | EXCAVATION WORK PERMIT | Corrected (old row 22 — “Landscaping” removed; utility locate added: key gap in a live station) |
| 12 | Working in Confined Spaces (incl. entry/exit) | 2 | 5 | 10 | Medium | Confined Space Permit; atmospheric test (O2, LEL, H2S, CO) before entry AND continuous monitoring; stand-by attendant at the opening with constant communication; rescue equipment (tripod/harness) at the entry point; no lone working (min. 2 persons); gas-free certificate for tanks/bays; SELV/PELV electrical equipment only; access & egress clearly defined; emergency exit kept clear; housekeeping inside. | 1 | 4 | 4 | Low | CONFINED SPACE WORK PERMIT | Corrected (old rows 9+14+15 merged; the copied scaffolding text replaced; stray template numbers removed) |
| 13 | Electrical Work — High Voltage (≥1000 V) | 3 | 5 | 15 | HIGH | Precautions implemented BY THE EXECUTING CONTRACTOR with coordination from plant operations (corrected: was “by the customer”): 5 safety rules — isolate, lock out / tag out, verify that equipment cannot be re-energized, ground and short-circuit, maintain isolation from adjacent energized elements; Electrical Work Permit + EKAD; only electrically trained and certified personnel; insulated tools; arc-rated PPE; rescue plan. | 1 | 4 | 4 | Low | ELECTRICAL WORK PERMIT + EKAD | Corrected (S4 was undefined — now 5=Death; residual S kept at 4: isolation does not remove all residual arc/flash potential) |
| 14 | Working Near Energized Points (LV/HV busbars & conductors) | 3 | 5 | 15 | HIGH | Live-area map agreed with plant operations; barricades + signage restricting the energized zone; safety distance calculated BEFORE work; energy shutdown requested where possible (then the 5 rules apply); otherwise Energized Work Permit; employees trained on energized points; no raising of held materials overhead. | 2 | 4 | 8 | Medium | ELECTRICAL WORK PERMIT | Corrected (old row 23; residual stays MEDIUM — live-plant proximity cannot be reduced to Low without verified isolation) |
| 15 | Fire / Explosion (incl. oil storage & gas areas) | 2 | 5 | 10 | Medium | Hot Work Permit; remove combustible loads; fire watch assigned; appropriate extinguishing agents on site (CO2/D-class); gas cylinders ≥3 m apart or fire-resistant partition; hoses disconnected from supply when work interrupted; NO SMOKING in oil storage area; LEL monitoring in gas areas. | 1 | 4 | 4 | Low | HOT WORK PERMIT | Corrected (old row 18; S raised — gas/fuel-plant potential is fatal) |
| 16 | Gas / Fuel System Commissioning (natural gas) | 2 | 5 | 10 | Medium | Gas-free certificate before work; continuous LEL monitoring (fixed + portable detectors tested); Hot Work Permit + fire watch; mechanical and electrical isolation verified; procedure per OEM; qualified personnel only; ■■ TO BE COMPLETED ■■ confirm whether gas commissioning is within this JHA’s scope. | 1 | 4 | 4 | Low | HOT WORK PERMIT + GAS-FREE CERT | NEW |
| 17 | Work in a Live Operating Plant (interface with operations) | 3 | 5 | 15 | HIGH | Joint JHA with plant operations; live-area map updated and displayed; permanent barricades around live parts; agreed outage/blackout windows; single PTW authority defined (client or SE — ■■ TO BE COMPLETED ■■ open item per PTW review); daily interface meeting; no work inside the live zone without a permit. | 2 | 4 | 8 | Medium | Per client PTW system | NEW (residual permanently MEDIUM — inherent to an operating station) |
| 18 | Hot Surfaces (turbine, piping, steam during commissioning) | 3 | 4 | 12 | Medium | Surface temperature check before work; “HOT SURFACE” signage; temporary insulation; exclusion zone; heat PPE (EN 407); no work near surfaces >60 °C without treatment; gas-free before touching pipework. | 2 | 3 | 6 | Low | N/A | NEW (risk group 18 was not ticked in Rev 0) |
| 19 | Heat Stress (extreme climate — Basra) | 4 | 4 | 16 | HIGH | Work/rest cycle in extreme heat (e.g. 45/15 min — confirm per applicable standard); mandatory hydration points; shaded/air-conditioned rest areas; acclimatization program for new arrivals; medical screening; schedule work in cool hours (night/early morning); monitor heat index; training on heat-illness symptoms; first-aid response for heat stroke. | 2 | 3 | 6 | Low | N/A | NEW (risk group 21 was not ticked — the key site risk in Basra) |
| 20 | Noise Exposure (turbine / compressor areas) | 3 | 3 | 9 | Medium | Noise survey + dosimetry; hearing protection (EN 352) in designated zones; signage; limit exposure time; periodic hearing tests. | 2 | 2 | 4 | Low | N/A | NEW |
| 21 | Dust / Fume Exposure (cutting, grinding, welding fumes, chemicals) | 2 | 3 | 6 | Low | Wet cutting where possible; local exhaust ventilation; FFP3 respirator (EN 149 — corrected: template mislabeled EN 136); periodic air monitoring. | 1 | 2 | 2 | Low | N/A | NEW (risk group 15 ticked in Rev 0, no row existed) |
| 22 | Moving Machine Parts & Equipment (entrapment, impact) | 3 | 3 | 9 | Medium | Lock out machinery before maintenance; guards in place; no reaching into moving parts; trained personnel only; exclusion during operation. | 2 | 2 | 4 | Low | LOTO | NEW (risk group 16 ticked in Rev 0, no row existed) |
| 23 | Ergonomic Strain / Manual Handling | 3 | 2 | 6 | Low | Team lift above ■■ TO BE COMPLETED ■■ kg (confirm limit); mechanical aids; lifting-technique training; task rotation. | 2 | 2 | 4 | Low | N/A | NEW (risk group 6 ticked in Rev 0, no row existed) |
| 24 | Use of Chemical Substances | 2 | 3 | 6 | Low | SDS available on site for every chemical; labeled, segregated storage; use per SDS; chemical-specific gloves; spill kit at point of use. | 1 | 2 | 2 | Low | N/A | NEW (risk group 12 ticked in Rev 0, no row existed) |
| 25 | Oil / Hazardous Material Spill (turbine oil, hydraulic oil, chemicals) | 2 | 3 | 6 | Low | Spill kits at turbine-oil and hydraulic points; containment bunds; trained spill response; report to client EHS; SDS on site. | 1 | 2 | 2 | Low | N/A | NEW |
| 26 | Inadequate / Missing PPE (re-framed hazard) | 2 | 3 | 6 | Low | Hazard re-framed: PPE is a CONTROL; the hazard is non-use or damage. Minimum set: helmet EN 397 (chin strap at height), work shoes EN 345 S3, glasses EN 166, gloves EN 388, FFP3, high-visibility; task-specific: EN 361 harness (heights), arc clothing + electrical gloves (electrical), EN 407 (heat). Monthly inspection with records; issue by notification & receipt; replace on demand. | 1 | 3 | 3 | Low | PPE assignment form | Corrected (old row 8 re-framed) |
| 27 | Hand / Power Tools & Measuring Equipment | 3 | 3 | 9 | Medium | Tools appropriate to the task; visual inspection before use; guards fitted; safety devices never bypassed; low-vibration tools or scheduled breaks; tools secured against slipping; adjacent areas shielded from flying particles; hydraulic/pneumatic lines safely routed with whip checks; SDS followed for hydraulic fluids; trained personnel only. | 2 | 2 | 4 | Low | N/A | Corrected (old row 10 — duplicate sentences removed) |
| 28 | Waste Management | 3 | 2 | 6 | Low | Waste sorted by group (metal, cable, wood, paper, hydrocarbon); disposed in the designated waste area; MOTAT records; licensed disposal; no on-site burning. | 1 | 2 | 2 | Low | MOTAT Records | Corrected (old row 19 — S 3→2; stray “&” removed) |
| 29 | Fire Evacuation Awareness | 2 | 5 | 10 | Medium | Instructions communicated per the Customer Emergency Procedure; evacuation routes and assembly points signed; MONTHLY evacuation drill (recorded); trained fire team; first-aid team trained. | 1 | 4 | 4 | Low | N/A | Corrected (old row 17; S raised — evacuation failure in a fuel/gas plant is fatal) |
| 30 | Emergency Response / Evacuation Failure | 2 | 5 | 10 | Medium | Emergency Action Plan aligned with the client and communicated to all employees; response teams identified and trained (first aid, fire, search & rescue, evacuation) with recorded training; contact list posted visibly and on the back of every ID card; first-aid & emergency kits content defined by the EHS specialist and technical authorities. | 1 | 4 | 4 | Low | N/A | Corrected (old row 24 re-framed: the plan is a control; the hazard is response failure) |
| 31 | Medical Evacuation / Delayed First Aid | 2 | 4 | 8 | Medium | Nearest hospital identified + response time documented: ■■ TO BE COMPLETED ■■; dedicated transport vehicle; first-aid kits in all vehicles; trained first-aiders on site; medevac plan agreed with the client. | 1 | 3 | 3 | Low | N/A | NEW |
| 32 | Security (theft, civil disturbance, travel risk — Iraq/Basra) | 3 | 4 | 12 | Medium | Security plan per client & SE requirements: site access control & perimeter; patrols; safe room; redundant communications (satellite/backup); travel control and escort; insurance; regional situation monitoring; ■■ TO BE COMPLETED ■■ confirm client security arrangements. | 2 | 4 | 8 | Medium | N/A | NEW (residual permanently MEDIUM — regional risk) |
| 33 | Wild Animal Bites / Vector-Borne Disease (corrected from “volatiles”) | 2 | 4 | 8 | Medium | Routine vaccination completed before travel + regional immunizations if required; vector control (screens, nets, still-water removal, chemical control by experts); behavioral training (food disposal, clothing, repellents); first-aid training (wash bite with soap & cold water 5 min, apply pressure, seek help via 112 immediately); hospital with snake antivenom identified: ■■ TO BE COMPLETED ■■. | 1 | 3 | 3 | Low | N/A | Corrected (old row 21 — “Panzer status”→vaccination, “volatiles”→vectors, copied driving sentence removed; residual now actually improves 8→3) |
| 34 | STOP Authority / Incident Management | — | — | — | n/a | REMOVED FROM THE RISK MATRIX (old rows 4 & 5): these are EHS management processes, not hazards — they cannot be scored L×S. Tracked as EHS program KPIs: stop-usage statistics, incident & near-miss reporting rate, investigation closure, lessons learned. Incident reporting: immediate, no matter how minor; everyone supports the investigation and corrective/preventive actions. | — | — | — | n/a | N/A | Removed — moved to EHS program |

Removed/merged rows (old → new):  11 & 12 (duplicate) → 6  |  14 & 15 (duplicate + copied text) → 12  |  4 & 5 (management processes) → 34 (unscored)  |  7 (mismatched) → 9  |  16 → 4  |  17 → 29  |  18 → 15  |  19 → 28  |  20 → 5  |  21 → 33  |  22 → 11  |  23 → 14  |  24 → 30  |  8 → 26  |  9 → 12  |  10 → 27  |  1–3, 6 → 1–3, 13

STEP 3 — PPE & EQUIPMENT (corrected: references risk numbers instead of blanket “All”)

| Work category (risk refs.) | LOTO | Helmet EN 397 | Ear plugs EN 352 | Safety shoes EN 345 S3 | Mech. gloves EN 388 | Heat gloves EN 407 | Protective clothing | Eyewear EN 166 | Face shield | Harness EN 361 | Arc clothing | Electrical gloves | FFP3 (EN 149) | Hi-vis | Fire ext. | Other |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All work (baseline) | — | All | — | All | — | — | All | All | — | — | — | — | — | All | — | — |
| Electrical work (13, 14) | All | All | — | All | — | — | All | All | — | — | All | All | — | All | — | Insulated tools |
| Working at height (6, 7) | — | All (chin strap) | — | All | All | — | All | All | — | All | — | — | — | All | — | — |
| Confined space (12) | — | All | — | All | All | — | All | All | — | All | — | — | FFP3 | All | — | Gas detector, tripod |
| Hot work / gas (15, 16) | — | All | — | All | EN 407 | EN 407 | All | All | All | — | — | — | FFP3 | All | On site | LEL detector |
| Hot surfaces / commissioning (18) | — | All | — | All | EN 407 | EN 407 | All | All | All | — | — | — | FFP3 | All | On site | — |

STEP 4 — APPROVAL (re-signature required for Rev 1)

| Head of Project Manager | Quality Manager | Project Manager | EHS | Result |
| --- | --- | --- | --- | --- |
| Name: Uğur Akbulut Date: ____________ Signature: ____________ Phone: ■■ TO BE COMPLETED ■■ | Name: Abdurrahman Öğüt Date: ____________ Signature: ____________ Phone: ■■ TO BE COMPLETED ■■ | Name: Onur Özvatan Date: ____________ Signature: ____________ Phone: ■■ TO BE COMPLETED ■■ | Name: Doğukan Arandı Date: ____________ Signature: ____________ Phone: ■■ TO BE COMPLETED ■■ | SUITABLE / NOT SUITABLE All four signatures required. NOTE: the EHS approver must be independent of the work team (corrected conflict of interest). |

NOTE: In Rev 0 only 2 of 4 signatures were present (Akbulut and Özvatan unsigned; Öğüt and Arandı digitally signed 04.03.2026). All four signatures are required, with re-signature for Rev 1.

STEP 5 — COMMUNICATION TO EMPLOYEES (must be completed BEFORE work starts)

| Trainer / Sign | Training Area | Training Date |
| --- | --- | --- |
| ■■ TO BE COMPLETED after briefing ■■ | ■■ TO BE COMPLETED after briefing ■■ | ■■ TO BE COMPLETED after briefing ■■ |

| No | Name & Surname | Signature | No | Name & Surname | Signature |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | 11 |  |  |
| 2 |  |  | 12 |  |  |
| 3 |  |  | 13 |  |  |
| 4 |  |  | 14 |  |  |
| 5 |  |  | 15 |  |  |
| 6 |  |  | 16 |  |  |
| 7 |  |  | 17 |  |  |
| 8 |  |  | 18 |  |  |
| 9 |  |  | 19 |  |  |
| 10 |  |  | 20 |  |  |

Declaration: “I guarantee that I will follow the rules mentioned in the precautions part of the risk assessment form, and also under negative conditions I will stop working in a safe way, then inform my project manager.”

ANNEX A — PROJECT-LEVEL RISKS (start of the KAZ PP Project Risk Register — full version in the accompanying XLSX)

| ID | Project Risk | L | S | R | Rating | Owner | Mitigation (summary) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | Procurement / long-lead equipment delays (turbine parts, generator, transformers) | 3 | 5 | 15 | HIGH | ■■ TO BE ASSIGNED ■■ | Early orders, on-site expediting, alternative sources, schedule buffer |
| P2 | Grid interconnection & synchronization (grid code, protection coordination, reverse power, connection permit) | 3 | 4 | 12 | Medium | ■■ TO BE ASSIGNED ■■ | Early coordination with grid operator / Ministry, protection & sync studies, sync test plan |
| P3 | Interface with the operating plant: outage windows, production loss, operational liability | 3 | 4 | 12 | Medium | ■■ TO BE ASSIGNED ■■ | Agreed outage windows, sequenced work packages, production-liability insurance, daily interface meeting |
| P4 | Hot commissioning (turbine start, run-up, load) deviations | 3 | 4 | 12 | Medium | ■■ TO BE ASSIGNED ■■ | OEM procedures, hold points, qualified test team, gas safety plan |
| P5 | Permitting & customs (equipment import via Basra port, environmental permits) | 3 | 3 | 9 | Medium | ■■ TO BE ASSIGNED ■■ | Early applications, local agent, document checklist before shipping |
| P6 | Security & geopolitical risk (Basra) | 3 | 4 | 12 | Medium | ■■ TO BE ASSIGNED ■■ | Site security plan, insurance, travel policy, regional monitoring |
| P7 | Currency (IQD) & payment terms | 3 | 3 | 9 | Medium | ■■ TO BE ASSIGNED ■■ | Contract currency clause, hedging, clear payment schedule |
| P8 | Subcontractor performance / single-point dependency | 3 | 3 | 9 | Medium | ■■ TO BE ASSIGNED ■■ | Performance monitoring, KPIs, qualified backup contractors |
| P9 | Extreme summer weather impacting schedule | 4 | 3 | 12 | Medium | ■■ TO BE ASSIGNED ■■ | Schedule around work windows, night work, heat protocol |
| P10 | Schedule delay / liquidated damages | 3 | 4 | 12 | Medium | ■■ TO BE ASSIGNED ■■ | Critical-path management, float monitoring, weekly early warning |

Document status: DRAFT FOR REVIEW — Rev 1 (corrected), 14.09.2026. All ■■ TO BE COMPLETED ■■ placeholders must be filled before approval. Job-based hazard analysis and the work permit consist of 5 steps; work starts only after Step 5 is completed.