# KAZ POWER PLANT UPGRADE PROJECT — MANAGEMENT & STAFF EHS INDUCTION

# Rev. 01 — Proposed Full Rewrite (Slide-by-Slide Content)

**Document No.:** KAZ-EHS-IND-001 · **Rev.:** 01 · **Status:** For Review & Approval

**Prepared by:** EHS Manager (KAZ PP Upgrade) — Al-Mial (AMC)

**Reviewed by:** Siemens Energy EHSMIP · **Approved by:** Project Director / EHS Director

**Classification:** Restricted — Siemens Energy is a trademark licensed by Siemens AG

**Validity of this induction:** 12 months from the date of attendance (renewal required on expiry, or earlier after a serious incident or a change of hazards)

> **How to use this document:** every `SLIDE` block is the exact content to be placed on one PowerPoint slide. Text in `[ ]` is an instruction to the deck author (photo, diagram, or appendix reference). Arabic trainer notes are marked **ملاحظة**. Keep slide numbers and the footer `KAZ-EHS-IND-001 Rev.01 — Restricted — Page X of Y`.

## SLIDE 1 — Cover

**KAZ POWER PLANT UPGRADE PROJECT**

**Management & Staff EHS Induction**

Khor Al-Zubair Gas Turbine Power Plant — Basrah, Iraq

Client: Ministry of Electricity (MoE) & Basrah Gas Company (BGC) · Principal Contractor: Siemens Energy

Presented by: Al-Mial (AMC) EHS Department | Document No. KAZ-EHS-IND-001 | Rev. 01 | Date: ___

Approved by: Project Director / EHS Director / SE EHSMIP

**ملاحظة للمدرب:** يُذكر صراحة أن هذا الإندكشن جزء من متطلبات Siemens Energy، ولا يُغني عن PSI ولا عن تصريح العمل.

## SLIDE 2 — Emergency instructions for this session

If an emergency occurs during this induction: stop the session, follow the exit signs and proceed to the nearest **Assembly Point (Muster Point)** — location: ………

Alarm signal: continuous siren = evacuate · intermittent = stand by · all-clear = return to work

Mobile phones on silent (keep your phone available for emergencies)

Smoking is prohibited except in the designated smoking area (never in red/restricted zones)

Emergency numbers (see Slide 63): Site EHS Control Room — ……… · UHF Channel 1 · Ambulance 122 · Fire 115 · Police 104 · Siemens Energy Emergency +90 533 625 9494

## SLIDE 3 — Contents (must match the actual deck)

1. Objectives & validity · 2. Project description & site · 3. HSE policy & SE Zero Harm · 4. Roles & responsibilities · 5. General site rules · 6. Stop Work Authority · 7. Risk management (RA/SJA/LMRA/Toolbox/MoC) · 8. Permit to Work & LOTO · 9. Electrical safety & arc flash · 10. High-risk activities (height, lifting, excavation, confined space, hot work, tools) · 11. PPE · 12. Health, heat stress & first aid · 13. Environment, waste & chemicals · 14. Fire & firefighting · 15. Security, access & journeys · 16. Driving & mobile plant · 17. Emergency response & incident reporting · 18. Lessons learned · 19. Violations & penalties · 20. Assessment, card & sign-off

## SLIDE 4 — Objectives

At the end of this induction you will be able to:

State the project HSE policy and the **Siemens Energy Zero Harm** framework

Apply the general site rules and use your PPE correctly

Exercise **Stop Work Authority** and explain your protection from retaliation

Read a risk assessment, complete an **LMRA** and know when an **SJA** is required

Explain how the **Permit to Work (PTW)** and **LOTO** systems work on this site

Recognise the electrical, height, lifting, excavation, confined-space and hot-work hazards of KAZ

Respond correctly to a fire, an emergency, an injury, a spill or a security event

Report incidents, near misses, unsafe acts and conditions within the required timeframes

## SLIDE 5 — Validity, assessment and records

Induction validity: **12 months**; renewal on expiry; re-induction after a serious incident or a change of hazards

Knowledge check: **10 questions — pass mark 80%** (retraining required if below)

Your attendance and signature are recorded in the site training register

The induction card must be carried and shown on request — **the card does not replace the Permit to Work**

Minimum site age: **18 years**

## SLIDE 6 — Project description

The KAZ Power Plant Upgrade Project modernises and expands the existing **400/132/11 kV Air Insulated Switchgear (AIS)** facilities at the Khor Al-Zubair Power Plant, Basrah, Iraq.

Scope includes: new **400 kV and 132 kV bays** (circuit breakers, disconnectors, earthing switches, CTs, VTs, surge arresters, busbar extensions), new foundations, earthing grid enhancement, steel structures, and all control, protection, metering, telecom and SCADA/DAS cable works, including civil, mechanical, electrical installation, testing, commissioning and trial operation.

**Project duration: 13.5 months** · Working hours: see Slide 15

Client: **MoE & BGC** · Principal Contractor: **Siemens Energy** · Subcontractor: **Al-Mial (AMC)**

**The works are executed inside a LIVE power plant** — coordination with MoE Operations is mandatory for every activity.

## SLIDE 7 — Site plot plan, work areas and emergency facilities

[Insert simplified plot plan with bilingual legend]

Project working area · Mobilisation / laydown area (TSF) · Main gate · Warehouses · Camp & offices

**Emergency layer:** assembly points, first-aid points, fire points, eyewash, emergency exits, ambulance access route, nearest hospital

**ملاحظة للمدرب:** يجب تعريف أي اختصار يظهر في المخطط (مثل TSF) في مفتاح الخريطة.

## SLIDE 8 — Project EHS organisation & contacts

[Organisation chart with names/functions]

Project Director / Site Manager — overall responsibility for EHS

**SE EHSMIP / SE EHS Manager** — governance, audits, verification, Stop Work authority

**EHS Manager (Contractor)** — daily supervision, permits, inspections, training

**EHS Officers** — one per work area/shift (No EHS supervision = No work)

**Area Authority / Operations Manager (MoE)** — authorises work in live operational areas

**AEP / Isolation Authority** — energy isolation, LOTO, energisation

First aiders, fire wardens, gas testers — posted in every area

## SLIDE 9 — HSE policy

**Siemens Energy EHS Policy (Annex 1 — SE Standard 000050) and the KAZ Project EHS Plan apply to everyone on this site.**

Project commitment: **Zero Harm** — zero injuries, zero occupational illnesses, zero environmental incidents, zero security breaches.

Project target: **Zero H1 = ZERO**.

Safety takes precedence over progress and cost

Everyone has a duty to protect themselves, colleagues, the environment and assets

Health, safety and environmental protection are conditions of employment on this project

Contractor policy: Al-Mial (AMC) — **“Do it safely, or don’t do it”** (supporting the SE framework)

## SLIDE 10 — Siemens Energy Zero Harm framework

**Zero incidents is achievable · Health and safety – no compromises · We take care of each other**

Zero Harm behaviours: take the time to assess risk and plan; talk about safety before and during the work; stop and re-assess when things change; follow the rules; intervene and never walk by

Zero Harm Essentials: the Stop principle, safe systems of work, competency, risk assessment, PTW, PPE, emergency preparedness

**ملاحظة للمدرب:** اربط كل مبدأ بمثال من موقع KAZ (مثلاً: إيقاف رافعة بسبب الريح، أو إيقاف عمل بدون قفل شخصي).

## SLIDE 11 — What Zero Harm means for you today

Plan your task before you start it — never improvise around a hazard

Use the right permit for the right job

Verify your PPE before every task

Report every near miss, unsafe act and condition — reporting is not blame

Intervene politely when you see risk — it is expected, not optional

## SLIDE 12 — Your responsibilities

**Everyone** shall: take care of own and colleagues’ safety; comply with site rules, permits and instructions; report hazards, incidents and near misses; use PPE correctly; attend training and toolbox talks; never work under the influence of alcohol or drugs; stop unsafe work.

**Supervisors / Superintendents** additionally: brief the crew, complete LMRA/SJA, verify permits and certifications, control the work area, verify controls before restart, ensure only competent personnel perform the task.

**Managers / Staff** additionally: provide resources, verify compliance in their areas, lead safety walks, hold contractors accountable, ensure EHS is the first agenda item in every meeting.

## SLIDE 13 — Contractor and subcontractor obligations

Each contractor/subcontractor works to its own EHS system enriched with Siemens Energy, Client and KAZ project requirements

Every contractor appoints a competent **Contractor EHS Representative**

Contractors provide: risk assessments, method statements, permits, competency certificates, equipment certificates, monthly man-hours and incident data

Siemens Energy may **stop any work**, exclude or ban individuals for violations — at the contractor’s cost

Where local/legal requirements differ from SE requirements, **the stricter requirement applies**

## SLIDE 14 — General site requirements (1)

Complete the **site induction + PSI (Personal Safety Instruction)** before starting any work

**No work without a valid Permit to Work**; no work without EHS supervision (**No EHS supervision = No work**)

Complete an **LMRA** before every task; attend the daily **Toolbox Talk**

Complete an **SJA** for high-risk tasks (heights, lifting, confined space, live electrical, hot work…)

Read and understand the risk assessment and method statement for your task — ask if anything is unclear

Never start work in an area you have not been authorised to enter (red zones, live areas)

## SLIDE 15 — General site requirements (2)

Working hours: **06:00–15:00, 6 days/week** (subcontractor hours approved by SE); maximum 11 hours daytime / 7.5 hours night work, one rest day in every 7 — no fatigue driving or working

**Minimum PPE at all times on site** (see Slide 43)

Only competent, certified persons operate plant and equipment

Keep your workplace **clean and tidy — “Clean everything – always!”**

Report every near miss, unsafe act, unsafe condition and incident (Slide 62)

## SLIDE 16 — Electrical safety rules (1)

1. Treat every electrical circuit as **live until proven dead**

2. Isolate the source before work; **LOTO** to prevent re-energisation

3. **Prove the tester before and after testing** — then prove dead

4. Only an **Authorized Electrical Person (AEP)** may isolate, de-isolate or energise; each worker applies a **personal lock**

5. Never work on live equipment unless formally authorised and only under an **Electrical Hot/Commissioning Permit**

6. Use insulated tools suitable for the voltage; verify calibration and condition

7. Maintain safe approach distances at all times (Slide 31–32)

8. Keep panels and access routes clear; report damaged equipment, cables or covers immediately

**Golden Rule: ISOLATE — LOCK OUT — TAG OUT — TEST BEFORE TOUCH**

## SLIDE 17 — Electrical safety rules (2) — Arc flash and switching

Arc flash risk assessment exists for this site: respect the **Arc Flash Boundary**, use the specified PPE category, never bypass barriers or interlocks

Only competent, appointed persons may switch, ground or energise (Operations Manager, HV Safety Supervisor, Switching Supervisor, AEP)

Work within **30 m of a high-voltage system** requires agreement with the plant owner and physical demarcation

**Stop all work during thunderstorms** — cranes, lifts, scaffolding and installation work must be suspended (lightning and induced voltage)

Report any contact with a conductor, any flash and any damaged insulation immediately — do not touch the casualty before isolation

## SLIDE 18 — Safe distances (project-specific, metric)

[Table approved by SE EHSMIP and the Client — to be inserted]

| Nominal voltage | Minimum approach distance (m) | Arc flash boundary (m) | Notes |

|---|---|---|---|

| 11 kV | … | … | Limited / restricted approach per NFPA 70E + CV |

| 132 kV | … | … | Client operational rules apply |

| 400 kV | … | … | Client operational rules apply — highest risk on this site |

**Rules:** no crane, MEWP, ladder, scaffold, excavator or vehicle may enter the protected zone; barriers and signage must remain in place until the work is completed; **look up and look around before moving any plant or load**.

## SLIDE 19 — Prohibited items and behaviours

Alcohol, narcotics and other intoxicating substances — **zero tolerance**; random testing enforced; refusal or a positive result = immediate removal and permanent ban

Firearms and ammunition — prohibited for all personnel (except authorised security personnel)

**Smoking — only in designated smoking areas**; never in red zones or near flammables

Weapons, fighting, harassment, discrimination, theft, sleeping and horseplay — zero tolerance

Photography and video recording **anywhere on site** — prohibited without written authorisation from the Site EHS/Security Manager

Mobile phones — not in red zones/restricted areas; never while driving (including hands-free)

Removal of any material, tool, equipment or waste requires a **Material Gate Pass**

## SLIDE 20 — Stop Work Authority

**Anyone** may stop any task — employee, contractor, visitor — when there is imminent danger, a rule violation, a missing control or a change in conditions

Steps: 1) Stop the activity · 2) Intervene directly with those involved · 3) Escalate to the supervisor/area authority · 4) Verify and implement controls · 5) Restart only when authorised · 6) Record and feed back

**You will not be punished, prosecuted or harassed for stopping work** — this is a Siemens Energy commitment

Work remains stopped until the controls are in place and the area authority authorises a restart

Siemens Energy reserves the right to stop work and suspend any activity that is unsafe

## SLIDE 21 — Risk management: RA, SJA and LMRA

**Risk assessment (RA):** read it, understand it, follow the controls; never work outside its scope

**SJA (Safe Job Analysis):** required for high-risk/non-routine tasks; prepared with the crew before work starts

**LMRA (Last Minute Risk Assessment):** short check before every task — What can hurt me? What has changed? Are controls in place? If in doubt — stop and ask

**Management of Change:** if the method, equipment, personnel or site conditions change, work stops and the assessment is revised and approved

## SLIDE 22 — Daily safety routine

Start of shift: toolbox talk → LMRA → permit verification → PPE check → equipment pre-use check

During work: supervision present, housekeeping maintained, barriers and signage in place, no lone working on high-risk tasks

End of shift: close/endorse permits, restore isolations per the LOTO register, secure the area, hand over outstanding tasks in writing

Shift handover for isolated work: joint re-verification of permit conditions, isolation status and zero-energy state with signatures

## SLIDE 23 — Permit to Work (1) — system and roles

**No permit = No work.** The PTW system is controlled by Siemens Energy; for work on live/operational plant the Client (MoE/BGC) permit system takes precedence

Roles: **Permit Issuer (SE)** · **Permit Holder (contractor supervisor)** · **Area Authority (operations)** · **Isolation Authority/AEP** · **EHS Reviewer** · **Authorized Gas Tester**

All permits require: approved JSA/method statement, verified controls on site, briefed working party, valid certificates and equipment inspections

EHS reviews and audits permits; technical approval belongs to the Area Authority; isolation authority belongs to the AEP

## SLIDE 24 — Permit to Work (2) — permit types and validity

| Permit | Typical validity |

|---|---|

| General Cold Work | 7 days with daily endorsement |

| Electrical Cold Work | 7 days with daily endorsement |

| Electrical Hot Work & Commissioning | Daily / each shift |

| Energy Isolation (LOTO) | Daily / each shift |

| Hot Work (welding, cutting, grinding) | Daily / each shift |

| Excavation & Trenching (> 30 cm) | 7 days with daily endorsement |

| Lifting (all lifts; detailed plan for critical lifts) | Daily / each shift |

| Work at Height (≥ 1.8 m) | Daily / each shift |

| Confined Space Entry | Daily / each shift |

## SLIDE 25 — LOTO and energy isolation

Cover **all** energy sources: electrical, mechanical, pneumatic, hydraulic, thermal, stored energy

Sequence: notify → isolate → lock & tag → release stored energy → verify **zero energy** → work

Every worker applies their **own personal lock**; keys are kept in the lockbox under the AEP’s control

**PTW validity ≠ LOTO validity** — expiry of a permit does not release an isolation; only the AEP de-isolates through the controlled process

Never remove another person’s lock or tag — this is a dismissible offence

## SLIDE 26 — Work at height

**Fall protection is required from 1.8 m** (or where a fall hazard exists)

Use approved scaffolding, platforms or MEWPs; harness with **double lanyard and shock absorber**, anchored to a certified point (> 22 kN), **100% tie-off**

Scaffolding: only erected/modified by certified scaffolders, **green tag** valid, inspected before each shift/after alteration, no work on red-tagged scaffolds

Ladders: short-duration work only, secured, 3 points of contact, no carrying loads, no working from the top steps

Barricade the area below; tether tools; never throw materials; protect openings and edges

## SLIDE 27 — Lifting operations

**All lifts require a permit**; critical lifts (load > 75% of capacity, multi-crane, over live plant/energised equipment, non-routine) require a detailed engineered lifting plan

Requirements: certified crane and lifting tackle (third-party certificates), competent operator, rigger and banksman, exclusion zone, taglines

Outriggers fully extended on timber pads on firm ground; verify ground bearing capacity

**No lifting in wind above 32 km/h** or in poor visibility; never lift over personnel or live equipment

Load control: no side pulls, no shock loading, nobody under the load

## SLIDE 28 — Excavation and trenching

A permit is required for any ground penetration deeper than **30 cm**

Obtain the signed **underground utility clearance** (electrical/cables/earthing grid) before digging — **hand-dig trial pits near services**

Protective systems (shoring, benching or sloping) are mandatory for excavations deeper than **1.2 m**

Spoil heaps set back at least **1.0 m** from the edge; barricade and sign all excavations; ladders for egress every 7.5 m

Inspect after rain or any ground movement; keep plant and vehicles away from the edge; never enter an unprotected trench

## SLIDE 29 — Confined spaces

Entry only with a **Confined Space Entry Permit**, trained entrants/attendants and a rescue plan

Atmospheric testing before entry and continuously during entry: **O₂, LEL, H₂S, CO** by an Authorized Gas Tester

Ventilation, isolation (LOTO) and rescue equipment in place; the attendant stays at the entry at all times

**Never enter to rescue** without breathing apparatus and the rescue team — most confined-space fatalities are would-be rescuers

Excavations are classified as confined spaces only by risk assessment, not automatically

## SLIDE 30 — Hot work and fire prevention

**Hot Work Permit** required for welding, cutting, grinding, soldering, thermal spraying or any spark/flame/heat above 100 °C

Requirements: competent certified welder, fire blanket containment, **10 m clearance** of combustibles, dedicated **Fire Watch during the work and for 30 minutes after**, minimum two extinguishers on site, gas cylinders secured upright, separated (fuel/O₂) and capped

Notify the area and the fire point before starting; check for hidden combustibles; screens to protect others

**Never** perform hot work where a flammable atmosphere may exist or near fuel/chemical areas without a gas test

## SLIDE 31 — Hand and power tools

Use the right tool for the job; inspect before use; remove defective tools from service immediately

Portable electrical tools: 230 V with **30 mA RCD**, armoured/undamaged cables, earthed (verify earthing measurement), no taped joints

Angle grinders: guard in place, correct disc for the material, no disc over-speed, wear **full face shield (EN 166)** + safety glasses + gloves

Secure the workpiece; avoid cutting towards your body; no loose clothing, no jewelry, no rings, tie back long hair

Compressed air is not for cleaning clothes or skin

## SLIDE 32 — Manual handling and ergonomics

Assess the load: weight, size, grip, centre of gravity, distance and destination

Use mechanical aids or get help; bend knees, keep the load close, no twisting while lifting

Plan breaks and vary tasks; report any pain early — do not wait for it to become an injury

Team lifting with a leader and clear commands for heavy or awkward loads

## SLIDE 33 — PPE: mandatory minimum on site

Helmet with **chin strap** + high-visibility vest

**Flame-retardant coverall** (visibility class 1)

Safety shoes with toe protection and puncture resistance (**S3/S1P**)

Safety glasses (ANSI/EN certified)

Protective gloves (task-specific) and hearing protection in high-noise areas (**> 85 dB(A)**)

Additional PPE is specified by the task risk assessment: arc-rated PPE (Category 2/4), face shield, insulated gloves, full-body harness, chemical-resistant PPE, respiratory protection

**Respiratory protection is mandatory where dust/vapour is identified (e.g., FFP3) and requires face-fit testing before use**

PPE must be in good condition and a good fit — damaged PPE is replaced free of charge

## SLIDE 34 — Health, heat stress and hydration

Heat stress is managed by **WBGT monitoring** (monitoring starts when ambient temperature exceeds **40 °C**) with published work/rest cycles per task intensity

**Acclimatisation** applies to all new arrivals and to anyone returning from leave: reduced workload and increased rest in the first days

Drink water regularly (**1.0–1.2 litres per hour** for light work in high heat); use oral rehydration salts; shaded, ventilated rest areas are provided

**Lone working is prohibited** on high-risk tasks — use the buddy system

Warning signs: headache, dizziness, cramps, nausea, confusion, hot dry skin, fainting → move to shade, cool the body, give fluids if conscious, call the medic/UHF Ch.1 immediately — **heat stroke is a medical emergency**

Ramadan and summer peak-hour arrangements are published separately and must be followed

## SLIDE 35 — First aid and medical emergency

First-aid points, trained first aiders, eye-wash stations, stretchers and the site clinic locations are marked on the plot plan

Everyone must know: the nearest first-aid point, the site medic, the emergency number and the route to the nearest hospital

Rules: do not move a casualty unless there is immediate danger; do not touch an electric-shock casualty before isolation; keep the airway open; start **CPR/AED** if trained and the casualty is not breathing; preserve the incident scene

Meet the ambulance at the main gate and escort it in; report the emergency to the EHS Control Room immediately

Fitness for work: valid medical certificate, report any illness/injury/medication that affects your ability to work safely

## SLIDE 36 — Environment: our obligations on site

Prevent pollution — spills, leaks, dust, noise and waste are all reportable events

**No spill of oil, fuel, chemical or concrete washwater to soil or drainage** — use drip trays, secondary containment, funnels and spill kits

Dust suppression on unpaved roads and cutting operations; noise and vibration controlled by selection of plant and work methods

**Asbestos and PCBs are prohibited on site**; suspected material must be reported and treated by certified personnel

Protect flora, fauna and heritage: no uncontrolled clearing, no hunting, no littering

Report environmental incidents (leaks, spills, uncontrolled emissions) immediately — they are classified as EHS incidents

## SLIDE 37 — Waste management

Segregate at source: general waste · recyclables (metal, cable, paper, plastic) · **hazardous waste** (oil-contaminated, batteries, filters, chemicals, e-waste) · construction/demolition waste

Hazardous waste is stored in labelled, covered, bunded areas and moves only with **MOTAT records and a licensed transporter**

Never burn waste, never mix hazardous with general waste, never dump into trenches, watercourses or empty land

Material and waste removal requires a **Material Gate Pass**

Keep all waste areas tidy, marked and away from drains

## SLIDE 38 — Chemicals and hazardous substances

No chemical may be brought to site without a **Safety Data Sheet (SDS)** and registration in the electronic substance register

Read the label and the SDS; use the specified PPE; never decant into unlabelled containers

Store flammables separately in approved cabinets, in limited quantities; keep oxidisers away from fuels

Eye-wash and emergency shower must be accessible where chemicals are handled

Report all contact, inhalation or ingestion immediately

## SLIDE 39 — Fire classes and extinguishers (corrected)

Fire classes (state the standard used):

**Class A** — wood, paper, cloth, rubber, some plastics → water, foam (DCP acceptable but less effective)

**Class B** — gasoline, kerosene, paint, propane → foam, DCP, CO₂

**Class C (NFPA)** — energised electrical equipment → **CO₂ or clean agent + de-energise the supply** (never water)

**Class D** — metals (magnesium, titanium, sodium) → **special metal powder (graphite/NaCl) — NEVER water or standard DCP**

**Class K/F** — cooking oils and fats → **wet chemical** (camp kitchen)

[Images of each extinguisher type]

## SLIDE 40 — Using a fire extinguisher (PASS)

**P**ull the pin · **A**im at the base of the fire · **S**queeze the handle · **S**weep side to side

Hold the extinguisher upright; stand between the fire and your escape route, back to the exit

For CO₂: hold the **insulated horn**, never the metal parts (cold burn risk)

**Only fight a small fire if you are trained and have a clear escape route** — otherwise raise the alarm and evacuate

Call the alarm first; evacuate if the fire grows, produces dense smoke or involves gas cylinders

Extinguishers are inspected monthly and serviced annually — never use a defective or missing extinguisher: report it

## SLIDE 41 — Security and access control

Site and camps are protected by the Iraqi Police and site security; all persons, belongings, vehicles and deliveries may be searched on entry and exit

Your **Site ID / Gate Pass** must be worn visibly at all times; access without it is refused

Visitors: pre-approval 24 h in advance, photo ID deposited at the gate, escorted by the host at all times; no manual work, no plant operation and no photography

Lost or damaged ID card: report immediately (replacement fee applies)

Security personnel are authorised to carry weapons — cooperate calmly and follow their instructions

Do not resist or interfere with security checks; theft, unauthorised access, suspicious persons or vehicles must be reported at once

Red zones (high-voltage areas) are marked — no entry without authorisation

## SLIDE 42 — Journey management and site movements

Movement only with prior authorisation; plan every journey (route, time, weather, checkpoints, contacts)

Use authorised drivers and vehicles only; no unauthorised passengers; no material transport without a gate pass

Seat belts for all occupants, at all times, in every vehicle

Rest at least every two hours; no driving when tired, ill or after long shifts; **no night driving without approval**

Report any accident, breakdown or security incident on the road immediately

Emergency contact details must be carried in every vehicle

## SLIDE 43 — Driving and mobile plant on site

Site speed limit **5 km/h**; obey all traffic signs, signals and marshals

**No mobile phone use while driving — including hands-free**; no eating, no distraction

Reverse parking only, in designated areas; **reversing requires a trained banksman** — vehicle/plant movements must be controlled by a flagman

Pre-use checks every day and valid third-party inspection certificates; vehicles display the site vehicle pass, flashing beacon and reversing alarm

Pedestrians use designated walkways; keep a safe distance from mobile plant; make eye contact with operators

Driving is restricted in adverse weather (fog, heavy rain, dust storms) — non-essential journeys are postponed

## SLIDE 44 — Emergency response & evacuation

| Signal | Action |

|---|---|

| Continuous siren | Stop work, isolate your equipment, proceed to the nearest assembly point |

| Intermittent siren | Stand by, prepare to evacuate, await instructions |

| All-clear | Return to work only when authorised |

Steps: raise the alarm → stop the job and isolate energy → proceed to the assembly point → **roll call (head count)** → report any missing person → follow announcements → do not leave or return until instructed

Fire wardens/area marshals are responsible for sweeping their area and confirming the head count, including visitors and subcontractors

Do not use vehicles, do not block emergency routes, keep the gate clear for emergency services

Evacuation for security incidents (threat, intrusion, civil disturbance) follows the same assembly-point principle with security instructions

## SLIDE 45 — Emergency classification and notification

| Level | Description | Notification |

|---|---|---|

| **Level 1** | Minor injury/health effect, minor damage or minor environmental impact, manageable on site with site resources | Supervisor → EHS → Site Manager (immediate verbal + written within 24 h) |

| **Level 2** | Significant event manageable locally with support from the Company or local authorities (major injury, local damage, local impact) | EHS → SE EHSMIP → Project Director + Client (immediate) |

| **Level 3** | Major event/crisis beyond the contractor’s control, with broader implications, requiring Company/corporate resources | Project Director → SE Corporate/Regional Hub + Client (immediate) |

**Life-threatening injury, fatality, high-potential incident, major environmental release and security crisis are escalated immediately — never treated as a locally managed event.**

Notification: verbal **immediately**, written notification **within 24 hours**, investigation report within **7 days** (unless an extension is agreed), CAPA status reported weekly.

## SLIDE 46 — Emergency contacts (posted and verified)

| Contact | Number / channel |

|---|---|

| Site EHS Control Room / emergency line | ……… (24/7) |

| UHF radio | Channel 1 |

| Site medic / first aiders | ……… |

| AMC emergency numbers | 0785-285-7404 · 0786-162-9706 |

| Siemens Energy emergency | +90 533 625 9494 |

| Ambulance | **122** |

| Civil Defence (Fire) | **115** |

| Police | **104** |

| Nearest hospital / Medevac | ……… |

**Rule:** call the site emergency line first; keep the line free; state your name, company, exact location, what happened, number of casualties and remaining hazards.

## SLIDE 47 — Incident, near miss and unsafe condition reporting

Report **immediately**: all injuries (including first aid), near misses, unsafe acts, unsafe conditions, property damage, environmental releases, security events

Report through your supervisor and the EHS Department; incidents are recorded in the **Siemens Energy EHS reporting tool (Enablon)**

Timelines: verbal immediately · written notification within **24 hours** · investigation report within **7 days** · CAPA follow-up weekly

Investigation: an investigation team is formed by the Contractor, the Company and subcontractors; root causes and corrective/preventive actions are documented

**Never** withhold information, adjust the scene or delay reporting; reporting is protected — no blame for honest reporting

Monthly man-hours and incident data must be submitted by every contractor

## SLIDE 48 — Learning from incidents

Lessons Learned and safety alerts are issued after incidents and high-potential near misses

Each lesson triggers: risk assessment review, toolbox talk, and verification that controls are effective

Siemens Energy may conduct **Eye on Safety** reviews and its own investigations

If you are involved in or witness a serious incident: preserve the scene, cooperate fully with the investigation, and provide a written statement

**Your experience prevents the next accident — share it.**

## SLIDE 49 — Violations and penalties

| Violation | 1st offence | 2nd offence | 3rd offence / critical |

|---|---|---|---|

| Speeding (> 5 km/h) | Warning + 2-day vehicle ban | 1-week vehicle & driver ban | Permanent site ban |

| PPE non-compliance | Work stoppage + written warning | Contractor fine | ID cancellation + site removal |

| Red zone / LOTO breach | Work stoppage + 1-week suspension | — | Permanent site expulsion |

| Lost/damaged ID card | Replacement fee | — | — |

| Positive alcohol/drugs test | Immediate permanent ban | — | — |

| Unauthorised material removal | Confiscation + investigation | — | — |

| Unauthorised photography | Media deletion + warning | — | Permanent site ban |

Siemens Energy may stop work, suspend, exclude or ban any individual or contractor for violations of EHS requirements, site rules or the law.

## SLIDE 50 — Knowledge check (10 questions)

[Sample questions — insert blank answer sheet]

1. What is the site speed limit and the reversing rule?

2. What permit is required for work at 2.5 m height?

3. When is an excavation classified as requiring a protective system?

4. What are the four gases tested before confined-space entry?

5. What is the maximum wind speed for lifting operations?

6. Which extinguisher for an energised electrical panel fire?

7. What are the three reporting deadlines after an incident?

8. What must you do before touching any electrical conductor?

9. What are the correct emergency numbers in Iraq? 10. What is your right if you see an unsafe act?

**Pass mark: 80%.**

## SLIDE 51 — Induction card and acknowledgement

Collect your numbered **induction card** — valid **12 months**, must be carried and shown on request

Sign the attendance and acknowledgement register before leaving this session

The induction card **does not replace** the Permit to Work, PSI, Safe Start or task-specific training

Your feedback on this induction is welcome — report content gaps to the EHS Department

**Acknowledgement text:** “I attended the KAZ Management & Staff EHS Induction, I understood its content, and I accept that compliance with safety rules, permits and Stop Work Authority is a condition of my access to this site.”

## SLIDE 52 — Closing

**Everyone goes home safely — every day.**

Siemens Energy **Zero Harm** · “We take care of each other”

Questions? Contact the EHS Department (Site EHS Control Room — UHF Channel 1)

Thank you

# Annexes to be attached to the deck

1. **Annex 1 — Permit quick-reference card** (permit type, issuer, validity, prerequisites).

2. **Annex 2 — LMRA pocket card** (4 questions, stop criteria).

3. **Annex 3 — Emergency card** (numbers, assembly points, hospital route — bilingual).

4. **Annex 4 — PPE matrix** (task vs PPE).

5. **Annex 5 — Attendance, acknowledgement and assessment form.**

6. **Annex 6 — Arabic/Turkish summary of critical slides** (electrical safety, PTW/LOTO, heat stress, emergency, reporting).

# Approval block

| Role | Name | Signature | Date |

|---|---|---|---|

| Prepared by — EHS Manager (Contractor) | | | |

| Reviewed by — SE EHSMIP | | | |

| Reviewed by — Construction/Site Manager | | | |

| Approved by — Project Director | | | |

| Approved by — EHS Director | | | |