| SE GT SU Kaz Power Plant Upgrade Project Risk Analysis |  |  |  |
| --- | --- | --- | --- |
| Job-based hazard analysis is performed in A, B, C, Small projects and, by decision of EHS managers, in all projects. |  |  |  |
| Job-based hazard analysis and the work permit consist of 5 steps: Steps 1, 2, 3, 4 and 5. |  |  |  |
| Step 1 |  |  |  |
| EXPLANATION: The existing general potential risks and hazards in the work area are defined. The relevant risk is ticked. 26 risk groups were defined. |  |  |  |
| Risk Groups | Relevant Work Permits | Work permits related to the risk groups in Step 3 |  |
| 1- Electric shock or exposure to electric arc |  | 1 | ELECTRICAL WORK PERMIT |
| 2- Fall from height |  | 2 | WORKING AT HEIGHT PERMIT |
| 3- Falling object from height |  | 3 | WORKING AT HEIGHT PERMIT |
| 4- Hot work (oxy-LPG, oxy-acetylene, LPG, torch) |  | 4 | HOT WORK PERMIT |
| 5- Welding (electrode, aluminum) |  | 5 | HOT WORK PERMIT |
| 6- Ergonomic strain / manual handling |  | 6 | JHA |
| 7- Hazardous waste |  | 7 | MOTAT Records |
| 8- Use of mobile electrical panel and extension cable |  | 8 | ELECTRICAL WORK PERMIT |
| 9- Use of crane, hoist, forklift, compressor, personal carrier, etc. |  | 9 | CRANE WORK PERMIT, LIFTING PLAN |
| 10- Use of construction machinery (excavator, truck) |  | 10 | EXCAVATION WORK PERMIT |
| 11- Angle grinder use |  | 11 | JHA |
| 12- Use of chemical substances (ethyl alcohol, acid, base, epoxy, etc.) |  | 12 | JHA |
| 13- Waste generation due to activity (cable, metal, paper, hydrocarbon, etc.) |  | 13 | MOTAT Records |
| 14- Explosive areas |  | 14 | JHA |
| 15- Exposure to dust, vapor, gas, welding fumes, chemical and biological agents |  | 15 | JHA |
| 16- Moving machine parts or work equipment (entrapment, impact, etc.) |  | 16 | JHA |
| 17- Slip and trip hazards on the ground |  | 17 | JHA |
| 18- Contact with extremely cold or hot surfaces |  | 18 | JHA |
| 19- Working near energized open busbars and conductors in low-voltage systems |  | 19 | ELECTRICAL WORK PERMIT |
| 20- Working in high-voltage system (≥1000 V) |  | 20 | ELECTRICAL WORK PERMIT, EKAD |
| 21- Environmental factors, working in extremely hot & cold climate |  | 21 | JHA |
| 22- Hand tool use (hammer, cutting tools, etc.) |  | 22 | JHA |
| 23- Use of scaffolding, ladder and platform |  | 23 | WORKING AT HEIGHT PERMIT |
| 24- Insufficient lighting and ventilation |  | 24 | CONFINED SPACE WORK PERMIT |
| 25- Working in confined or restricted space |  | 25 | CONFINED SPACE WORK PERMIT |
| 26- Use of pressure vessel |  | 26 | CONFINED SPACE WORK PERMIT |
| Step 2 |  |  |  |
| Existing work steps should be written one by one and work equipment to be used specifically for the job should be specified on a separate line for construction equipment and subjected to hazard assessment. |  |  |  |
| Severity and likelihood are entered for each step. The severity/likelihood table uses a 5x5 matrix. |  |  |  |
| According to the severity and likelihood result, the Risk score should be entered as low, medium and high. |  |  |  |
| Control measures for each written step must be evaluated, and the risk score should decrease in line with the measures taken/to be taken. |  |  |  |
| Step 3 |  |  |  |
| Relevant PPE is selected to protect against the risks mentioned in Step 2. |  |  |  |
| Under each PPE in Step 3, the related risk number from Step 2 is referenced to confirm. |  |  |  |
| Step 4 |  |  |  |
| This is the approval section of the job hazard analysis. |  |  |  |
| The site manager/field supervisor and OHS specialist conduct the final review and mark the appropriate section. |  |  |  |
| The OHS specialist informs the field supervisor/site manager as needed that work permits are required for the relevant work steps. |  |  |  |
| If 'Not Suitable' is indicated, this hazard assessment must be redone. Work cannot start during this time. |  |  |  |
| Step 5 |  |  |  |
| The approved job hazard analysis must be communicated to employees. In this section, the person performing or supervising the work conveys the content of Steps 2 and 3 to employees. |  |  |  |
| Employees acknowledge by writing their name, surname and signature in the spaces provided to confirm they accept the items in Steps 2 and 3. |  |  |  |
| Work starts after Step 5 is completed. |  |  |  |

SE-FO-0023

Rev: 0

ortamda bulunmaktadır.

Her Hakkı Saklıdır .Firmaya Özel

Siemens Energy, Siemens AG tarafından lisanslanmış tescilli bir ticari markadır	1

|  |  | Job Hazard Assessment (JHA) |  |  |  |  | Document No: | 1 |  |  | Date: 04.03.2026 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Subcontractor |  |  |  | Personnel Tot.: |  |
| Activity : Kaz Power Plant Upgrade Project (Constraction, instalation, Test and commitiong of SS extantion) |  |  |  |  |  |  | Used Vehicles: Manlift, Crane, Truck, Excavator |  |  | Used Equipment’s: Hand Tools, |  |  |
|  |  |  |  |  |  |  |  |  |  | Power Tools, |  |  |
| Location : Basrah Iraq |  |  |  |  |  |  |  |  |  | Lifting Equipment’s, Test Equipment’s |  |  |
| Work Area : Khur Alzubair Power Station |  |  |  |  |  |  |  |  |  |  |  |  |
| Team Member | Uğur Akbulut, Onur Özvatan, Abdurrahman Öğüt, Doğukan Arandı |  |  |  |  |  |  |  |  |  |  |  |
| NOTE: If after risk evaluation and with all the necessary controls in place; the risk rating (R) is HIGH, the EHS department are to be contacted for guidance on the task/process. |  |  |  |  |  |  |  |  |  | POLICE | FIRE STATION | AMBULANCE |
| SIEMENS ENERGY EMERGENCY CONTACT NUMBER +90 533 625 9494 |  |  |  |  |  |  |  |  |  | 112 |  |  |
| STEP 1 / ADIM 1 | HAZARD / POTANTIAL RISK GUIDE |  |  |  |  |  |  |  |  |  |  |  |
| HAZARD / POTANTIAL RISK GUIDE | Contact with electric shock or arc electric |  | x | The use of electrical panel and electrical cable | x | Contact with dust, steam, gas, welding smoke, biological & chemical substances |  |  | Working with overtemp & extreme cold, environmental factors |  |  |  |
|  | Falling from height |  | x | The use of cranes, chain hoist, forklift, compressors, personal lift, etc. | x | Moving part of the machines or work equipment |  | x | The use of hand tools |  |  | x |
|  | Piece falling from height |  | x | The use of construction equipment (scoop, truck) | X | Slipping points on the ground |  | x | The use of scaffolding, stairs, platform |  |  | x |
|  | Termal cutting (oxy lpg , oxy acetylene ,LPG, torch |  |  | The use of spiral grinding engine | X | Contact with extremely cold or hot surfaces |  |  | Inadequate lighting and ventilation |  |  | x |
|  | Welding (Electrodes, aluminum) |  |  | The use of chemical substances (ethyl alcohol, acid, base, epoxy etc.) | x | Near working around low voltage conductive and energized opened bare |  | X | Working with closed and circumscribed area |  |  | x |
|  | Ergonomic strain / lifting heavy |  | x | Produced waste as a result of activity (cable,metal,paper,hydrocarbons.etc) | x | Working with high voltage ( ≥1000 V ) |  | X | The use of pressure vessel |  |  |  |
|  |  |  |  |  |  | Others |  |  | Others |  |  |  |
|  | Hazardous Waste |  | X | Explosive Area |  | .......................................................................... .......................................................................... .......................................................................... .......................................................................... |  |  | .............................................................................................................................. .............................................................................................................................. .............................................................................................................................. ......................... |  |  |  |

Severity

| STEP 2 | THIS DOCUMENT IS RECORDED AND STORED BY THE DOING THE JOB |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Persons Affected: (e.g. Operators, maintenance, visitors, public etc.): |  |  |  | Handling: For Siemens Energy internal and subcontractor use only. Review: If there is a change in working methods and conditions, the form must be renewed. |  |  |  |  | Likelihood |  |  |  |  |
|  |  |  |  |  |  |  |  | 1 | 1 2 1 2 |  | 3 3 | 4 4 | 5 5 |
| The Risk Assessment |  | Severity (S) |  | Likelihood of harm (L) |  |  | Risk Score (R) = S x L |  |  |  |  |  |  |
| - Identify and record for each hazard/risk identified the appropriate rating for severity and likelihood without control. - Identify and record the control measures to be used then repeat the assessment with the controls in place. |  | 1:No Injury 2:Minor Injury 3:Lost time Injury 5:Death or Permanent Disability |  | 1:Very Unlikely 2:Unlikely 3:Likely 4:Very Likely 5:Certain |  |  | 1 to 6 = Low Risk (L) (Acceptable, Monitor & Review) 8 to 12 = Medium Risk (M) (Acceptable With Controls) >12 = High Risk (H) Unacceptable | 2 3 4 5 | 2 4 3 6 4 8 12 5 10 15 |  | 6 9 | 8 12 16 20 | 10 15 20 25 |
| No | Pathway of activity / |  | Assessment High (H) Medium (M) Low (L) |  |  | Precautions |  | Assessment High (H) |  |  |  |  |  |
|  |  |  | Likelihood (L) | Severity (S) | Risk Score(R) |  |  | Likelihood (L) |  | Severity (S) |  | Risk Score(R) |  |
| 1 | BEFORE WORK |  | 4 | 3 | 12 | 1-Make sure you have a valid assembly visa before going to the country for work 2-A competent person must be appointed for the specific task. For certain jobs (crane operator, forklift operator, welder, scaffold builder, electrician, mechanical maintenance man, etc.) the necessary vocational training and qualification documents should be requested. Unauthorized personnel should not work outside of the job description. - Make sure that you have a health report approved by the doctor for the last year or valid in your country. - Customer safety induction training and SE EHS field induction training should be taken before starting to work at the construction site. Learn the emergency management plan and actions from the customer. (Nearest hospital, timing, etc.) - Siemens Energy and the customer risk assessment must be properly read and understood before starting work. In addition, the training attendance report must be signed by each employee. - Unexpected earthquake, fire, explosion, etc. Keep your passport and some local cash in case of need. - The project site has a lot of risks within the scope of the work area, personal protective equipment should be provided for each employee. - High visible clothing - Chin-attached helmets - Head lamp - Work shoes with ankle protection *Ear protection *Safety glasses - Protective work gloves *Protective dust mask (FPP3) - Disposable overalls - Daily working hours cannot exceed 11 hours during the day and 7.5 hours at night, excluding rest periods. - Seven days of uninterrupted work cannot be done, one day of rest is obligatory within seven days. - The annual overtime period cannot exceed 270 hours. |  | 4 |  | 1 |  | 4 |  |
| 2 | GENERAL SITE WORK |  | 4 | 3 | 12 | - Fill the "LMRA" form before starting work and get the necessary work permit from the client. - When you see the risks that may affect you at the project site, inform the customer and ask for action. - Notify the project manager of the situation if the client does not take any action. |  | 4 |  | 1 |  | 4 |  |

| No | Pathway of activity / | Assessment High (H) Medium (M) Low (L) |  |  | Precautions | Assessment / High (H) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Likelihood (L) | Severity (S) | Risk Score(R) |  | Likelihood (L) | Severity (S) | Risk Score(R) |
| 3 | DRIVING - TRANSPORT TO SITE | 3 | 3 | 9 | - Everyone must have a valid driver's license for the type of vehicle they drive and be medically fit to drive, in accordance with local regulations. - Everyone is expected to plan their journeys taking into account distance, road conditions, travel time and weather conditions. - Everyone must obey all local traffic regulations and speed limits while driving. - No one should drive while under the influence of drugs or alcohol. - Everyone should wear a seat belt at all times. - No one should use mobile phones or other devices that cause distraction while driving, including in hands-free mode - this is unsafe and unacceptable. - Eating, drinking, setting up a radio or GPS device, etc. It is expected to minimize the risk of distraction. - No one should drive when tired or suffering from fatigue. - Everyone should take adequate breaks to drive at least every two hours. If you feel tired, stop and rest. - Anyone who orders and uses external personal transportation services and rents a vehicle must ensure that they meet this Zero Accident Culture behavior and the “Vehicle Safety” Principle. - Everyone should ensure that the loads carried are within the vehicle weight and load limits, and that the loads carried are properly secured before departure. - While driving, you should be careful about the possibility of wild or domestic animals on the road. - The vehicle should be used considering weather conditions such as rain, snow, fog, icing. - Tires suitable for seasonal conditions should be used (such as winter tires in cold and rainy weather, summer tires in hot weather) | 3 | 1 | 3 |
| 4 | GENERAL SITE WORKS (STOP) | 4 | 3 | 12 | - Everyone is empowered to apply Stop any time there is a potential risk of harm or non-compliance with Zero Harm Behaviors, Essentials, local Zero Harm programs, or legal requirements. - Taking into consideration the severity of the circumstances, stop! may be issued by anyone as follows: *Immediate direct verbal Stop! usually involving minor non-compliance, including unsafe acts or conditions that can be easily corrected on the spot without delay. *Immediate notification of Stop! to the responsible person in charge of the work activity or workplace condition for immediate corrective action. - Everyone experiencing a Stop! notification from another individual is expected to receive it in a constructive way and as a kind reminder of our Zero Harm Principle that “We Take Care of Each Other.” - Everyone must respect the Stop!, and it shall remain in force until required corrective measures have been put in place. - When in doubt how to resolve a situation leading to a Stop!, everyone must consult their managers and verify the applicability and implementation of required measures. | 4 | 1 | 4 |
| 5 | INCIDENT MANAGEMENT | 4 | 3 | 12 | - Everyone needs to know the Siemens Energy definitions and procedures for reporting incidents and near misses. - Everyone should ensure that any incident or near miss they are involved in or witnessed is reported to their manager or supervisor without delay. - Anyone who experiences an injury, no matter how minor, or who experiences sudden pain from a work activity should speak up without delay and report the situation to their manager or supervisor. - Anyone who has been involved in or witnessed an incident or near miss will support the investigation process and contribute to cause analysis, lessons learned and closure of the incident. - Everyone should be familiar with the corrective and preventive measures required following the investigation process and be fully aware of any changes in procedures or risk assessments covering business activities. - Everyone should support the implementation of corrective and preventive measures by proactively checking that everyone they work with is fully aware of changes. | 4 | 1 | 4 |

| No | Pathway of activity / | Assessment High (H Medium (M) Low (L) |  |  | Precautions | Assessment / High (H) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Likelihood (L) | Severity (S) | Risk Score(R) |  | Likelihood (L) | Severity (S) | Risk Score(R) |
| 6 | HAZARD DUE TO ELECTRICAL CURRENT, HIGH VOLTAGE | 4 | 4 | 16 | - The following precautions will be taken by the customer during Electrical Work Compliance with 5 safety rules: - Isolate - Lock Out Tag Out - Confirm that equipment cannot be re-energized - Grounding and short circuit - Maintain isolation with adjacent energized elements - Work permit should be taken. - Safety instructions must be followed. - Use only the tools appropriate for the activity. - Use shielded extra low voltage or safety extra low voltage in confined areas - Wear appropriate personal protective equipment - Only electrically trained and certified employees should work in electrical work. | 4 | 1 | 4 |
| 7 | Field Activities Trips/Falls/Injuries | 4 | 3 | 12 | - Materials being transported must be stacked in an organized manner. Materials should be stacked in separate areas according to their size and weight, ensuring they do not tip over or collapse. - Materials should be transported using appropriate vehicles. - Slings used for material transport must undergo visual inspection. Worn slings should not be used. - Do not stand under the load while lowering materials with a crane. Do not lift loads exceeding the crane's capacity. - Material transport and service vehicles should not be operated by anyone other than authorized personnel. - First aid kits and fire extinguishers must be available in the vehicles. Work should not be carried out in rainy, foggy, or heavy weather; weather reports should be checked before work begins." | 4 | 1 | 4 |
| 8 | PPE & Equipment | 4 | 3 | 12 | "At the start of work, all personnel's PPE assignment forms are checked using the personnel on-the-job control form. PPE deliveries should be made in the form of notification and receipt. PPE should be checked at least once a month and the checks should be recorded. If employees detect any malfunction or damage to their personal protective equipment, they should inform their supervisor and ensure it is replaced. Minimum PPE; - Helmet EN 397 (chin strap for teams working at height) - Work shoes EN 345 S3 or S1P (with sole and toe protection), Safety glasses EN 166 (anti-fog), Gloves EN 388 (for employees), EN361 safety harness should be used for work at height. - In addition, appropriate personal protective equipment (safety harnesses, welding masks/gloves, dust masks, etc.) according to the nature of the work should be delivered and recorded on the PPE assignment form. - Company logo work The outfit should be worn." | 4 | 1 | 4 |

| No | Pathway of activity / | Assessment High (H) Medium (M) Low (L) |  |  | Precautions | Assessment / High (H) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Likelihood (L) | Severity (S) | Risk Score(R) |  | Likelihood (L) | Severity (S) | Risk Score(R) |
| 9 | WORKING IN CONFINED SPACES(inadequate evacuation and rescue possibilities, poor communicationto the outside, risk of panic attacks, poor ventilation conditions,insufficient room to move) | 4 | 3 | 12 | - Before starting work, the necessary work permit must be obtained. - Perform a visual inspection of tagging and locking for correct application before entering confined spaces - Use only safe extra low voltage (SELV) or shielded extra low voltage (PELV) electrical equipment. - Assign a watchman on the entrance manhole and maintain constant contact with the watchman. - Check the atmosphere (oxygen concentration) before starting work, keep rescue equipment ready if necessary - Maintain order and cleanliness in confined spaces during maintenance work - It is forbidden to work alone; Must have at least 2 employees 8-Appropriate persons should be assigned to the job. - Respirator should be used if necessary - Ambient air measurement should be performed before entering the enclosed space. - When accessing the pier, only the access point provided for this purpose should be used. - Access and exit points should be clearly defined. 13-Emergency exits must be kept open at all times. | 4 | 1 | 4 |
| 10 | WORKING WITH HAND TOOLS/POWER TOOLS /MEASURING EQUIPMENTS | 4 | 3 | 12 | - Only use tools which are appropriate or intended for the activity - Visual inspection before use - Safety instructions in the instruction manual shall be observed - Wear appropriate personal protective equipment - Use low-vibration equipment or schedule sufficient breaks - Secure tools in such a way that slipping away during operations prevented - Shield adjacent work areas if injuries to others are possible due to flying particles or slung parts check inspection tag - Safety instructions in the instruction manual shall be observed - Only use tools which are appropriate or intended for the activity - Check to ensure that safety equipment is intact - Use only appropriately trained and qualified personnel - Lines (electrical, pneumatic, compressed and process air) shall be safely routed and secured - If hydraulic fluid is used, follow the instructions in the safety data sheet or the operating instructions - Wear appropriate personal protective equipment - Use whip check when using hydraulic and pneumatic systems. | 4 | 1 | 4 |
| 11 | HAZARD DUE TO FALLING | 4 | 3 | 12 | - Installation of scaffolding or barriers, only access scaffolding that has been inspected and approved - Observe work instruction on use of PPE to prevent falling! - Use PPE to prevent falling | 4 | 1 | 4 |

| No | Pathway of activity / | Assessment High (H) Medium (M) Low (L) |  |  | Precautions | Assessment /High (H) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Likelihood (L) | Severity (S) | Risk Score(R) |  | Likelihood (L) | Severity (S) | Risk Score(R) |
| 12 | HAZARD DUE TO FALLING | 4 | 3 | 12 | - Installation of scaffolding or barriers, only access scaffolding that has been inspected and approved - Observe work instruction on use of PPE to prevent falling! - Use PPE to prevent falling | 4 | 1 | 4 |
| 13 | HAZARD DUE TO SLIPPING, STUMBLING AND FALLING ON SAME LEVEL | 3 | 3 | 9 | - Lay cables outside the hazard zone - Materials and tools which are not required shall be removed from the work area - Traffic routes shall be kept clear - Keep the work area clean and tidy - Make sure that there is no deformation, lubrication or gap on the work area floor, and take the necessary precautions, if any. | 3 | 1 | 3 |
| 14 | HAZARD DUE TO ENTERING, EXITING Confined Space | 4 | 3 | 12 | - When accessing scaffolding, use only the access point provided for this purpose - Clearly define access and exit points - Keep emergency exits clear at all times Keep work area free from oil | 4 | 1 | 4 |
| 15 | HAZARD DUE TO ENTERING, EXITING Confined Space | 4 | 3 | 12 | - When accessing scaffolding, use only the access point provided for this purpose - Clearly define access and exit points - Keep emergency exits clear at all times - Keep work area free from oil | 4 | 1 | 4 |
| 16 | HAZARD DUE TO UNRESTRICTED AREA | 3 | 3 | 9 | - Barricade off work area if possible. - Keep personnel who are not required out of the hazardous area - Use only appropriately trained and qualified personnel | 3 | 1 | 3 |
| 17 | HAZARD DUE TO INSUFFICIENT KNOWLEDGE IN FIRE EVACUATION | 4 | 3 | 12 | Necessary instructions will be communicated to the staff in accordance with the Customer Emergency procedure. | 4 | 1 | 4 |

| No | Pathway of activity / | Assessment High (H) Medium (M) Low (L) |  |  | Precautions | Assessment / High (H) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Likelihood (L) | Severity (S) | Risk Score(R) |  | Likelihood (L) | Severity (S) | Risk Score(R) |
| 18 | HAZARD DUE TO FIRE OR EXPLOSION | 4 | 3 | 12 | - Only use tools which are suitable or intended for the activity - Remove hoses from combustible gases and oxygen or disconnect from supply point when work is interrupted - Maintain a distance of at least 3 meters between fuel gases and oxidation gases or provide for a fire-resistant partition - Safety instructions in the instruction manual shall be observed - Visual inspection before use - Remove fire loads, assign fire watches, provide for extinguishing agents - ATTENTION! Liquid gas cylinders (propane or butane) shall be stored at least 3 meters from ALL other types of gas cylinders - NO SMOKING in oil storage area - Wear appropriate personal protective equipment | 4 | 1 | 4 |
| 19 | Waste Management& | 3 | 3 | 9 | - Waste must be sorted according to waste group (metal, cable, wood, paper, etc.). - Waste must be sorted according to waste class and disposed of in the waste area. & | 3 | 1 | 3 |
| 20 | HAZARD DUE TO UNAUTHORIZED PERSONS/ OTHER SUBCONTRACTORS IN WORK AREA | 4 | 3 | 12 | - Mark areas and barricade off if necessary or set up warning signs. Inform subcontractors of risks | 4 | 1 | 4 |
| 21 | WILD ANIMAL BITES / EXPOSURE TO DISEASE-CARRYING VOLATILES | 2 | 4 | 8 | In mild injuries, the wound should be washed with soap and cold water for 5 minutes. The wound should be covered with a clean cloth. If there is serious injury and bleeding, the bleeding should be stopped by applying pressure to the wound with a clean cloth and support should be received immediately from 112. Panzer status for wild animals belonging to the study area should be learned from health centers. The worker should be transferred to the hospital in possible snake bites and it should be ensured that snake serum is intervened against snake bites. - Comprehensive information on common local carriers and diseases should be collected. If necessary, special expertise should be acquired at the first stage. - Personal behavioral training should be provided. (e.g. food disposal, clothing, fly repellents) - Mechanical vector control measures should be taken. (e.g. mosquito screens, mosquito nets) - Reproduction control should be done. (e.g. disposal of still water) - Chemical or biological vector should be controlled. (applied by experts) - If there is a deficiency in routine vaccination, it should be completed, if possible and necessary, additional immunization should be done. - While driving, you should be careful about the possibility of wild or domestic animals on the road. | 1 | 4 | 8 |
| 22 | Landscaping and Excavation Works& | 4 | 3 | 12 | - The licenses, maintenance records, and other legal documents for trucks and construction equipment (JCS, Loader, Excavator, etc.) to be used on site must be provided. - Drivers' personal documents must be submitted, and they must receive on-the-job training. - An additional observer must be present in the maneuvering and working area of trucks and construction equipment. - Workers must be prevented from entering the truck's blind spot, and training must be provided to workers on this subject. - No one should be near the truck when loading excavated material onto it. - Excavated material must be transported to the area indicated by the customer. Equipment must be available on trucks to prevent excavated material from spilling. | 4 | 1 | 4 |

| No | Pathway of activity / | Assessment High (H) Medium (M) Low (L) |  |  | Precautions | Assessment / High (H) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Likelihood (L) | Severity (S) | Risk Score(R) |  | Likelihood (L) | Severity (S) | Risk Score(R) |
| 23 | General field activities - Working near energized points | 4 | 3 | 12 | - Energized areas must be restricted using appropriate barriers, tape, etc. - Work must be performed using equipment that will not approach the safety distance (excavators, etc.). The safety distance must be calculated prior to work. - Energy shutdown must be requested for work within the safety distance, and the 5 safety rules must be applied. - Employees must be trained on energy points. - Employees must not raise materials they are holding upwards | 4 | 1 | 4 |
| 24 | Emergency situation& | 4 | 3 | 12 | - An emergency action plan should be organized and communicated to all employees. - Teams that will respond to emergencies should be identified (first aid team, fire team, search and rescue team, evacuation team, etc.) and trained, and the training provided should be recorded. - The list of names and contact information for emergency response teams must be posted in a visible location in the work area and mobilization area. - The contents of the first aid and emergency kits must be determined by the occupational safety specialist and technical authorities, procured, and kept on site. - Every worker on the construction site should be given an ID card. The back of the ID card should contain the project emergency numbers, and the front should contain the employee's identity information and the training they have received. | 4 | 1 | 4 |

| STEP 3 / | PPE & Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PPE & Equipment | LOTO | EN 397 Helmets | EN 352 Ear Plug | EN 345 S3 Safety Shoes | EN 388 Mechanical | EN 407 Heat-Tolerant Glove | Protective Clothing | EN 166 Eye wear | EN 166 Face Shield | EN 407 Thermo Safety Glove | EN 361 Full Body Harness | Electric Arc Protective Clothing | Electrical Safety Gloves | EN 136 Dust Mask | Reflective Vest | Fire Extinguisher | Other |
| Path the No | Electrical work if necessary | All | All | All | All |  | All | All |  |  | Working at height | Electrical work if necessary | Electrical work if necessary |  | All | All |  |
| STEP 4 / | HEAD of PROJECT MANAGER |  |  |  | QUALITY MANAGER |  |  |  | PROJECT MANAGER |  |  |  | EHS |  |  |  |  |
| Approval / Onay | Name Surname: UĞUR AKBULUT Date: Signature Phone |  |  |  | Name Surname: ABDURRAHMAN ÖĞÜT Date : Ogut Digitally signed by Ogut SignatureAbdurrah Abdurrahman man Date: 2026.03.04 11:53:38 +03'00' Phone |  |  |  | Name Surname: ONUR ÖZVATAN Date: Signature Phone |  |  | SUITABLE | Name Surname : DOĞUKAN ARANDI Date: Dijital olarak imzalayan Arandi Arandi DN: cn=Arandi Dogukan, Dogukan o=Siemens, Signature Dogukan Tarih 2026.03.04 11:18:39 email=dogukan.arandi@siemens- energy.com +03'00' Phone |  |  |  | SUITABLE |
|  |  |  |  |  |  |  |  |  |  |  |  | UNSUTABILE |  |  |  |  | UNSUTABILE |
| STEP 5 | TRAINER/SIGN |  |  |  |  |  |  |  |  | TRAINING AREA |  |  |  | TRAINING DATE |  |  |  |
|  |  |  |  |  |  |  |  | Name Surname |  | Signature |  |  |  |  |  |  |  |
| Personnel Signatures | 1. |  |  |  | 6. |  |  |  | 11. |  |  |  |  | 16. |  |  |  |
|  | 2. |  |  |  | 7. |  |  |  | 12. |  |  |  |  | 17. |  |  |  |
|  | 3. |  |  |  | 8. |  |  |  | 13. |  |  |  |  | 18. |  |  |  |
|  | 4. |  |  |  | 9. |  |  |  | 14. |  |  |  |  | 19. |  |  |  |
|  | 5. |  |  |  | 10. |  |  |  | 15. |  |  |  |  | 20. |  |  |  |
|  | I guarantee that i will follow the rules, mentioned on the precautions part of risk assesment form, and also under negative conditions i will stop working in a safe way,then inform my project manager |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Follow