|  | KAZ Power Plant Upgrade Project Khur Al-Zubair Power Station — Basra, Iraq KAZ-EHS-FRM-2026-007 • Rev. 0 • DRAFT FOR REVIEW (Pending Final Approval) |
| --- | --- |

CONTROLLED FORM PACK NIGHT CONCRETE POURING — EHS CHECKLIST PACK Pre-Pour Readiness, Certification, Electrical, Illumination, PPE, Medical, Barricading, TBT, Crew Fitness & Permit Closure Document ID: KAZ-EHS-FRM-2026-007 • Revision: 0 • Status: DRAFT FOR REVIEW (Pending Final Approval)

HOW TO USE THIS PACK

▪  Complete every checklist physically on site, at night, with the lighting on and the plant positioned as it will be used during the pour.

▪  Checklists C1–C7 must be fully signed BEFORE any concrete is placed; C8 and C9 before the crew enters the pour zone; C10 at shift end.

▪  Any “No” answer is a hold point: the pour shall not start until the item is closed or the work is re-scheduled to daylight.

▪  The signed pack is filed with the Permit to Work and referenced by KAZ-EHS-MS-2026-004 Sections 9 and 11.

▪  This pack is issued under the Siemens Energy KAZ Power Plant Upgrade Project EHS management system; the logo and document control above are part of the record.

CHECKLIST C1 — PRE-POUR NIGHT READINESS (MASTER)

A. DOCUMENTATION, PERMITS & CERTIFICATES

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Valid PTW issued/endorsed for this night shift, with all mandatory attachments |  |  |  |  |
| 2 | Method Statement KAZ-EHS-MS-2026-004 and JSA Tables 10.1/10.2 approved and on site |  |  |  |  |
| 3 | Concrete pump truck & placing boom — valid third-party inspection certificate sighted |  |  |  |  |
| 4 | Pump operator — valid third-party competency card sighted |  |  |  |  |
| 5 | All mixer drivers — valid heavy vehicle licences and third-party induction sighted |  |  |  |  |
| 6 | Site Medic certification (CPR / trauma / HV shock response) sighted |  |  |  |  |
| 7 | Ambulance inspection record and equipment check completed |  |  |  |  |
| 8 | Calibrated lux meter certificate valid; anemometer available |  |  |  |  |
| 9 | This checklist pack printed, and a clean copy available at the pour site |  |  |  |  |

B. CREW, FATIGUE & BRIEFING

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | 100 % night crew deployed — no day-shift worker continuing into the night |  |  |  |  |
| 2 | Every crew member recorded ≥ 8 h rest (Checklist C9) |  |  |  |  |
| 3 | Dedicated, fully rested Night EHS Officer present for the whole shift |  |  |  |  |
| 4 | Pre-shift fitness checks completed for all crew, drivers and operators |  |  |  |  |
| 5 | Day → Night handover log (FRM-2026-006) completed and signed |  |  |  |  |
| 6 | Night Toolbox Talk delivered; attendance signed (Checklist C8) |  |  |  |  |
| 7 | TBT covered approach distances, live energized zones, evacuation routes |  |  |  |  |
| 8 | Spotter, Banksman, Permit Holder and Medic named and present; radios tested |  |  |  |  |

C. BARRICADING, REBAR & ACCESS

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Pour area, open edges and pump perimeter enclosed with rigid hard barricades |  |  |  |  |
| 2 | Reflective tape and warning flashers fitted; illuminated signage at entry |  |  |  |  |
| 3 | Single controlled entry/exit manned; PTW access control enforced |  |  |  |  |
| 4 | 100 % of exposed vertical/horizontal rebar ends fitted with mushroom caps |  |  |  |  |
| 5 | Access pathways clear, lit and free of rebar, debris, standing water |  |  |  |  |
| 6 | Heavy-duty wheel stoppers at mixer discharge positions |  |  |  |  |
| 7 | Open excavations/pits/edges barricaded, lit, signed; lit ladders provided |  |  |  |  |
| 8 | Formwork, reinforcement, embedments released by Quality; pour sequence agreed |  |  |  |  |

D. PLANT, PUMP & TRAFFIC

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Pump on firm, level, compacted ground clear of excavations/services |  |  |  |  |
| 2 | Outriggers 100 % extended on pads; ground bearing confirmed |  |  |  |  |
| 3 | Boom maximum envelope measured, marked and briefed; clearance verified |  |  |  |  |
| 4 | Emergency stops, alarms, hydraulics, hoses, clamps inspected; line secured |  |  |  |  |
| 5 | Mixers one-way routed; holding area defined; speed limit briefed |  |  |  |  |
| 6 | Reverse alarms, lights, mirrors, brakes functional; baton working |  |  |  |  |
| 7 | Wind measured < 32 km/h (or manufacturer limit); monitored during shift |  |  |  |  |
| 8 | Washout pit/skip and spill kit in position; extinguishers at pump & boards |  |  |  |  |

E. ELECTRICAL, GROUNDING & RCD

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Pump body grounding connected directly to the station earth grid |  |  |  |  |
| 2 | Earthing continuity/resistance measured and recorded (C3) |  |  |  |  |
| 3 | 30 mA RCD on all outlets feeding vibrators, boards and temporary lights |  |  |  |  |
| 4 | RCD test button operated on every device; trip/reset confirmed and logged |  |  |  |  |
| 5 | Cables elevated off wet ground and fresh concrete on insulated hangers |  |  |  |  |
| 6 | Enclosures/connectors IP-rated; nothing standing in water |  |  |  |  |
| 7 | No damaged cables or taped repairs; defective equipment removed & tagged |  |  |  |  |
| 8 | Insulated rescue hook and insulating mats at the pour site |  |  |  |  |

F. ILLUMINATION & VISIBILITY

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | ≥ 100 lux at the concrete discharge / pouring point (C4) |  |  |  |  |
| 2 | ≥ 50 lux along access pathways and transit routes (C4) |  |  |  |  |
| 3 | Towers angled downward — no operator glare; no dark shadows |  |  |  |  |
| 4 | Battery-operated backup floodlights staged, charged and tested |  |  |  |  |
| 5 | High-intensity torches + whistles to Spotter, Banksman, EHS, Medic |  |  |  |  |
| 6 | Reflective marking on plant, outriggers, lines, barricades, vehicle tails |  |  |  |  |
| 7 | Class 3 high-visibility attire on 100 % of night personnel |  |  |  |  |

G. PPE & RESCUE GEAR

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Steel-toe rubber boots, chemical/rubber gloves, sealed goggles worn by handlers |  |  |  |  |
| 2 | Face shields / waterproof aprons / long sleeves for high-splatter tasks |  |  |  |  |
| 3 | Helmets with chin strap where fall or struck-by risk exists |  |  |  |  |
| 4 | Hearing protection, FFP3 masks, knee protection available as required |  |  |  |  |
| 5 | PPE inspected at TBT; damaged/contaminated PPE replaced |  |  |  |  |
| 6 | Illuminated first-aid kit at pour site; contents & expiry checked |  |  |  |  |
| 7 | Clean water / eyewash available; MSDS accessible |  |  |  |  |

H. MEDICAL & EMERGENCY

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Standby ambulance at the pour zone for the entire night shift |  |  |  |  |
| 2 | Dedicated ambulance driver on duty, remaining with the vehicle |  |  |  |  |
| 3 | Ambulance: oxygen, spinal board, resuscitation kit, working lights/siren |  |  |  |  |
| 4 | Certified Site Medic on duty on site for the whole shift |  |  |  |  |
| 5 | Evacuation route clear of all mixers, machinery and obstructions |  |  |  |  |
| 6 | Route/travel time/receiving point at Al-Zubair General Hospital confirmed |  |  |  |  |
| 7 | Alarm signal, muster point, contacts and radio channel briefed at TBT |  |  |  |  |
| 8 | Scenario responses briefed: shock, impalement, struck-by, fall, burn, boom failure |  |  |  |  |

I. ENVIRONMENT & HOUSEKEEPING

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Washout restricted to designated pit/skip away from drains & earth grid |  |  |  |  |
| 2 | Waste segregation in place; MOTAT records ready |  |  |  |  |
| 3 | Spill kit available; crew briefed on hydraulic/fuel spill response |  |  |  |  |
| 4 | Housekeeping completed and maintained; walkways and egress kept clear |  |  |  |  |

C1 READINESS AUTHORISATION — POUR MAY / MAY NOT COMMENCE

| Role | Name | Signature | Date / Time |
| --- | --- | --- | --- |
| Permit Holder / Civil Supervisor |  |  |  |
| Dedicated Night EHS Officer |  |  |  |
| Permit Issuer (Siemens Energy) |  |  |  |
| Area Authority (client / operations, where applicable) |  |  |  |
| Night Shift In-Charge / Construction Manager |  |  |  |
| Site Manager (Siemens Energy) |  |  |  |
| Concrete Pouring Subcontractor — Authorized Representative |  |  |  |

CHECKLIST C2 — PLANT & EQUIPMENT CERTIFICATION

| # | Certificate / Record to be Sighted on Site | Sighted (Y/N) | No. / Reference | Expiry | Verified By |
| --- | --- | --- | --- | --- | --- |
| 1 | Concrete pump truck — third-party inspection certificate (no. / expiry) |  |  |  |  |
| 2 | Placing boom — third-party inspection / load test certificate (no. / expiry) |  |  |  |  |
| 3 | Pump operator — third-party competency card (no. / expiry) |  |  |  |  |
| 4 | Mixer driver 1 — heavy vehicle licence + induction (no. / expiry) |  |  |  |  |
| 5 | Mixer driver 2 — heavy vehicle licence + induction (no. / expiry) |  |  |  |  |
| 6 | Mixer driver 3 — heavy vehicle licence + induction (no. / expiry) |  |  |  |  |
| 7 | Mixer trucks — vehicle inspection records (brakes, tyres, lights, reverse alarm) |  |  |  |  |
| 8 | Immersion vibrators — portable appliance test tags in date |  |  |  |  |
| 9 | Temporary distribution boards — inspection record and IP rating |  |  |  |  |
| 10 | Light towers — inspection record; fuel/cable condition |  |  |  |  |
| 11 | Lux meter — calibration certificate (no. / due date) |  |  |  |  |
| 12 | Anemometer — calibration / function check |  |  |  |  |
| 13 | Earthing kit — lead, clamps, continuity tester condition |  |  |  |  |
| 14 | Insulated rescue hook — certification / test date |  |  |  |  |
| 15 | Ambulance — inspection and roadworthiness record |  |  |  |  |
| 16 | Site Medic — certification (CPR / trauma / HV shock) validity |  |  |  |  |

CHECKLIST C3 — ELECTRICAL, GROUNDING & RCD

| # | Verification Item | Result / Reading | Pass / Fail | Initials | Time |
| --- | --- | --- | --- | --- | --- |
| 1 | Pump truck body grounding connected to the station earth grid (point identified) |  |  |  |  |
| 2 | Earthing continuity / resistance measured — record value and limit applied |  |  |  |  |
| 3 | RCD 1 — board/ID ______ : test button operated, tripped and reset |  |  |  |  |
| 4 | RCD 2 — board/ID ______ : test button operated, tripped and reset |  |  |  |  |
| 5 | RCD 3 — board/ID ______ : test button operated, tripped and reset |  |  |  |  |
| 6 | RCD 4 — board/ID ______ : test button operated, tripped and reset |  |  |  |  |
| 7 | All vibrator and tool cables elevated on insulated hangers |  |  |  |  |
| 8 | No cables, joints or sockets in water, laitance or fresh concrete |  |  |  |  |
| 9 | Enclosures IP-rated, dry, undamaged; no exposed conductors |  |  |  |  |
| 10 | No taped repairs or unauthorised extensions; defects removed and tagged |  |  |  |  |
| 11 | Light towers aimed downward; no glare; no dark shadows in work zone |  |  |  |  |
| 12 | Insulated mats and rescue hook available at the pour site |  |  |  |  |

CHECKLIST C4 — ILLUMINATION (LUX) SURVEY

Measured with a calibrated light meter at task / walking surface level. Re-measure after any change in tower position and at least once mid-shift.

| # | Measurement Point | Required | Measured (lux) | Pass / Fail | Action if Fail | Time |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Concrete discharge / pouring point (boom end / chute) | ≥ 100 lux |  |  |  |  |
| 2 | Working face — screeding / finishing area | ≥ 100 lux |  |  |  |  |
| 3 | Concrete pump set-up area and operator position | ≥ 100 lux |  |  |  |  |
| 4 | Rebar / formwork access pathways | ≥ 50 lux |  |  |  |  |
| 5 | Transit mixer route, turning and holding area | ≥ 50 lux |  |  |  |  |
| 6 | Mixer discharge / reversing position (banksman's view) | ≥ 50 lux |  |  |  |  |
| 7 | Emergency egress route to site gate | ≥ 50 lux |  |  |  |  |
| 8 | Muster point, ambulance station and first-aid point | ≥ 50 lux |  |  |  |  |
| 9 | Material storage, washout pit and waste area | ≥ 50 lux |  |  |  |  |
| 10 | Open edges, excavations and barricaded zones | ≥ 50 lux |  |  |  |  |

CHECKLIST C5 — PPE & RESCUE GEAR

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Class 3 high-visibility reflective attire issued to and worn by all personnel |  |  |  |  |
| 2 | Steel-toe rubber safety boots (EN ISO 20345 S5) worn by all concrete handlers |  |  |  |  |
| 3 | Heavy-duty chemical / rubber gloves (EN 388 / EN ISO 374) issued |  |  |  |  |
| 4 | Sealed safety goggles (EN 166) issued; face shields for high-splatter tasks |  |  |  |  |
| 5 | Helmets (EN 397) with chin strap where fall / struck-by risk exists |  |  |  |  |
| 6 | Hearing protection available near pump, vibrator and mixer |  |  |  |  |
| 7 | FFP3 dust masks available for cement handling and dry sweeping |  |  |  |  |
| 8 | Waterproof trousers / apron and knee protection for finishers |  |  |  |  |
| 9 | Insulated electrical rescue hook stationed at the pour site |  |  |  |  |
| 10 | Illuminated first-aid kit stocked, sealed and within expiry at the pour site |  |  |  |  |
| 11 | Clean water / eyewash station available; barrier cream provided |  |  |  |  |
| 12 | Contaminated or damaged PPE replaced immediately; spare stock on site |  |  |  |  |

CHECKLIST C6 — MEDICAL & EMERGENCY STANDBY

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Standby ambulance stationed directly at the concrete pouring area |  |  |  |  |
| 2 | Dedicated standby driver present and remaining with the ambulance |  |  |  |  |
| 3 | Oxygen cylinder charged and fitted |  |  |  |  |
| 4 | Spinal board present and accessible |  |  |  |  |
| 5 | Resuscitation kit present and sealed |  |  |  |  |
| 6 | Trauma dressings and burns kit present |  |  |  |  |
| 7 | Emergency lights and siren operational (tested) |  |  |  |  |
| 8 | Certified Site Medic on duty on site for the whole shift |  |  |  |  |
| 9 | Stretcher accessible at the pour site |  |  |  |  |
| 10 | Evacuation route to site gate clear of all plant and obstructions |  |  |  |  |
| 11 | Route, travel time and receiving point at Al-Zubair General Hospital confirmed |  |  |  |  |
| 12 | Hospital pre-notified of night works and crew numbers |  |  |  |  |
| 13 | Emergency contacts board displayed at the pour site and in the ambulance |  |  |  |  |
| 14 | Alarm signal and muster point briefed; headcount method agreed |  |  |  |  |

CHECKLIST C7 — BARRICADING, REBAR PROTECTION & ACCESS

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Pour area perimeter enclosed with rigid hard barricades (no soft tape) |  |  |  |  |
| 2 | Open edges hard-barricaded with handrails / fencing |  |  |  |  |
| 3 | Concrete pump perimeter enclosed and signed |  |  |  |  |
| 4 | Reflective tape fitted to all barricades, plant and outriggers |  |  |  |  |
| 5 | Warning flashers operating on the perimeter at night |  |  |  |  |
| 6 | Single controlled entry/exit point established and manned |  |  |  |  |
| 7 | PTW access control enforced; visitor log and escort rule in use |  |  |  |  |
| 8 | Illuminated / retro-reflective signage at the entry point |  |  |  |  |
| 9 | 100 % of exposed rebar ends fitted with approved mushroom caps |  |  |  |  |
| 10 | Spare mushroom caps kept at the pour site for replacements |  |  |  |  |
| 11 | Heavy-duty wheel stoppers installed at mixer discharge positions |  |  |  |  |
| 12 | Open excavations / pits barricaded, lit and signed; lit ladders provided |  |  |  |  |
| 13 | Access pathways and transit routes clear and lit |  |  |  |  |

CHECKLIST C8 — NIGHT TOOLBOX TALK RECORD

| Date / Time of TBT |  |
| --- | --- |
| Pour location / activity |  |
| Delivered by (Night EHS Officer) |  |
| PTW number |  |
| Language(s) used | ☐ English ☐ Arabic ☐ Other: ______ |

| # | Mandatory Topic — tick when covered | Covered (✓) |
| --- | --- | --- |
| 1 | Scope, sequence and planned volume of tonight's pour; joint locations |  |
| 2 | Safe approach distances; location of live energized zones and red zones |  |
| 3 | Marked boom envelope and the Spotter's stop signal; never stand under boom/hose |  |
| 4 | Barricaded perimeter, single controlled entry, PTW access control |  |
| 5 | Mixer routing, one-way circuit, reversing rules, banksman signals, wheel stoppers |  |
| 6 | Rebar mushroom caps, open edges, excavations, fresh-concrete exclusion |  |
| 7 | Electrical hazards: 30 mA RCDs, elevated cables, pump grounding, rescue hook |  |
| 8 | Wet concrete chemical burns: gloves, goggles, boots, immediate washing, eyewash |  |
| 9 | Illumination limits (100 / 50 lux), glare control, action if lights fail |  |
| 10 | Emergency evacuation routes, muster point, alarm, ambulance, hospital route |  |
| 11 | Fatigue, rest breaks, hydration and STOP WORK AUTHORITY |  |
| 12 | Environmental controls: washout pit, spill kit, waste segregation |  |
| 13 | Weather limits: wind > 32 km/h, lightning, dust storm — stop rules |  |
| 14 | Lessons learned from previous incidents / near misses on the project |  |

| # | Name / Surname | Company | Role | Signature |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |
| 16 |  |  |  |  |

CHECKLIST C9 — NIGHT CREW REST & FITNESS REGISTER

Minimum 8 hours rest before the shift for every individual. No person may work more than one shift in any 24-hour period.

| # | Name / Surname | Company | Role on Night Shift | Rest Start / End (≥ 8 h) | Fitness (✓) | Certs (✓) | Signature |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |

CHECKLIST C10 — POST-POUR, HOUSEKEEPING & PERMIT CLOSURE

| # | Verification Item | Yes | N/A | Verification Method | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Pump, boom, delivery lines and vibrators cleaned at the designated washout area |  |  |  |  |
| 2 | Residual concrete and washout water contained; disposed via approved route |  |  |  |  |
| 3 | Barricades, towers, boards and cables recovered or left safe, lit, barricaded |  |  |  |  |
| 4 | Temporary lighting left on over fresh concrete and open edges |  |  |  |  |
| 5 | Plant parked in designated area; keys controlled |  |  |  |  |
| 6 | Emergency egress lane restored and kept clear |  |  |  |  |
| 7 | Waste segregated and removed to approved storage; MOTAT records updated |  |  |  |  |
| 8 | PTW closed jointly by Permit Holder and Permit Issuer |  |  |  |  |
| 9 | Site condition, isolations and remaining hazards recorded at closure |  |  |  |  |
| 10 | Any incident / near miss recorded on form 000480 and in the Daily EHS Report |  |  |  |  |
| 11 | Night → Day handover log (FRM-2026-006) completed and signed |  |  |  |  |
| 12 | Fresh concrete, edges, openings and washout re-inspected at first light |  |  |  |  |

C10 CLOSURE SIGN-OFF

| Role | Name | Signature | Date / Time |
| --- | --- | --- | --- |
| Permit Holder / Civil Supervisor |  |  |  |
| Night EHS Officer |  |  |  |
| Permit Issuer (Siemens Energy) |  |  |  |
| Concrete Pouring Subcontractor Representative |  |  |  |